# Australian National Greenhouse Accounts Factors

For individuals and organisations estimating greenhouse gas emissions

2024

​© Commonwealth of Australia 2024

​Ownership of intellectual property rights

​Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

​Creative Commons licence

​

​Cataloguing data

​This publication (and any material sourced from it) should be attributed as: DCCEEW 2024, Australian National Greenhouse Accounts Factors, Department of Climate Change, Energy, the Environment and Water, Canberra, August. CC BY 4.0.

​This publication is available at www.dcceew.gov.au/climate-change/publications/national-greenhouse-accounts-factors

​Department of Climate Change, Energy, the Environment and Water

​GPO Box 3090 Canberra ACT 2601

​Telephone 1800 920 528

​dcceew.gov.au

​Disclaimer

​The Australian Government as represented by the Department of Climate Change, Energy, the Environment and Water has exercised due care and skill in the preparation and compilation of the information and data in this publication. Notwithstanding, the Commonwealth of Australia, its officers, employees, or agents disclaim any liability, including liability for negligence, loss how so ever caused, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying upon any of the information or data in this publication to the maximum extent permitted by law. No representation expressed or implied is made as to the currency, accuracy, reliability or completeness of the information contained in this publication. The reader should rely on their own inquiries to independently confirm the information and content on which they intend to act. This publication does not indicate commitment by the Australian Government to a particular course of action.

​Acknowledgement of Country

​We acknowledge the Traditional Owners of Country throughout Australia and recognise their continuing connection to land, waters and culture. We pay our respects to their Elders past and present.

Table of Contents

Australian National Greenhouse Accounts Factors	1

1	Introduction	5

2	Energy	7

3	Industrial processes and product use	30

4	Waste emissions	34

5	Agriculture	41

6	Land Use, Land Use Change and Forestry	42

Glossary	43

Appendix 1 Units and conversions	45

Appendix 2 Global Warming Potentials	47

Appendix 3 Methodology for calculating scope 3 greenhouse gas emissions from liquid and gaseous fuels	49

Appendix 4 Methodology for calculating electricity emission factors	53

Appendix 5 Methodology for calculating greenhouse gas emissions from waste	57

Appendix 6 Further Information	60

Appendix 7 NGER legislative amendments for the 2024–25 reporting year	62

Tables

Table 1 Indirect (scope 2 and scope 3) emission factors from consumption of purchased or acquired electricity	8

Table 2 Indirect (scope 2 and scope 3) emission factors from consumption of purchased or acquired electricity: market-based factors	9

Table 3: Indirect (scope 2) emission factors for electricity grids	13

Table 4 Direct (scope 1) and Indirect (scope 3) emission factors for the combustion of solid fuels - including certain coal based products	14

Table 5 Direct (Scope 1) emission factors for the consumption of gaseous fuels including liquefied natural gas	17

Table 6 Indirect (Scope 3) emission factors for the consumption of natural gas	18

Table 7 Indirect (Scope 3) emission factors for the consumption of ethane	19

Table 8 Direct (Scope 1) and indirect (scope 3) emission factors for the consumption of liquid fuels, including certain petroleum based products for stationary energy purposes	21

Table 9 Direct (scope 1) and indirect (scope 3) emission factors for the consumption of transport fuels in different transport equipment	25

Table 10 Indicative leakage rates for common refrigeration and air-conditioning equipment	30

Table 11 Global warming potentials of common refrigerants	30

Table 12 Direct (scope 1) emission factors for the calcination of carbonates	32

Table 13 Direct (scope 1) emission factors for use of carbonates in clay materials by state and territory	32

Table 14 Direct (scope 1) emission factors for the consumption of soda ash	33

Table 15 Waste mix methane conversion factors and emission factors	34

Table 16 Indirect (scope 3) waste emission factors for total waste disposed to landfill by broad waste stream category	35

Table 17 Emission factors by wastewater treatment type	37

Table 18 Direct (scope 1) emission factors for the incineration of waste	38

Table 19 Direct emission factors (scope 1) for biological treatment of waste	39

Table 20 Metric prefixes	45

Table 21 Energy conversion factors	45

Table 22 Unit equivalences	46

Table 23 Global Warming Potentials	47

Table 24 Blended refrigerants (many containing HFCS and/or PFCS)	48

Table 25 Transmission and distribution losses	55

Table 26 Simplification of emissions equations for wastewater treatment methods	57

Table 27 Simplification of emissions equations for waste incineration	57

Table 28 Simplification of emissions equations for waste incineration - municipal waste	57

Table 29 Simplification of emissions equations for biological treatment of waste	58

Table 30 Simplification of emissions equations for solid waste disposal	58

# Introduction

The 2024 National Greenhouse Accounts (NGA) Factors workbook has been prepared by the Department of Climate Change, Energy, the Environment and Water, and is designed to support individuals and organisations estimating their greenhouse gas emissions. The Department does not make any recommendations for the use of the methods contained within this document for any specific emissions accounting or reporting purposes.

This workbook (2024 NGA Factors Workbook) is not published for the purposes of reporting under the National Greenhouse and Energy Reporting (NGER) Scheme. Although drawing on the National Greenhouse and Energy Reporting (Measurement) Determination 2008, the methods described in the 2024 NGA Factors Workbook have a broad general application to the estimation of a range of greenhouse gas emissions inventories. Information and guidance on the NGER Scheme can be found at the Clean Energy Regulator website and a summary of latest amendments to the Measurement Determination can be found in Appendix 7.

The NGA Factors is subject to a system of continuous improvement. The Department welcomes feedback related to any of the content contained within the document. Feedback should be sent to nationalgreenhouseaccounts@dcceew.gov.au.

Links to other programs and initiatives related to greenhouse gas estimation and reporting are available in Appendix 6.

## Greenhouse gas emission sources

### Direct emissions

#### Scope 1

Direct (or scope 1) emissions are produced from sources within the boundary of an organisation and as a result of that organisation’s activities.  They are calculated at the point of emission release. These emissions include those from the following activities:

generation of energy, heat, steam and electricity, such as fuel combustion in generators;

manufacturing processes which produce emissions such as cement, aluminium and ammonia production;

transportation of materials, products, waste and people; such as the use of vehicles owned and operated by the reporting organisation;

intentional or unintentional GHG releases (fugitive emissions) such as methane emissions from coal mines, natural gas leaks from joints and seals; and

solid waste disposal and wastewater treatment including on-site waste management.

The default emission factors listed in this workbook have been estimated by the Department of Climate Change, Energy, the Environment and Water in conjunction with the production of the Australian National Greenhouse Accounts (ANGA). Unless otherwise stated, the methods for calculating emissions listed in this document are based on “Method 1” from the National Greenhouse and Energy Reporting (Measurement) Determination 2008.

This ensures consistency between inventories at company level and the emission estimates presented in the National Greenhouse Accounts. The methods used at the national level, and reflected in the factors reported here, are consistent with Intergovernmental Panel on Climate Change (IPCC) guidelines and are subject to international expert review each year. More information on the estimation methods employed in the National Greenhouse Accounts is available in the latest National Inventory Report.

### Indirect emissions

Indirect emissions are emissions generated in the wider economy as a consequence of an organisation’s activities but which are physically produced by the activities of another organisation. There are two classes of indirect emissions; electricity and other.

#### Electricity (scope 2)

Scope 2 emissions are indirect emissions which occur as a result of activities that generate electricity, heating, cooling or steam that is consumed by an organisation but which is generated outside that organisation’s boundaries. They are physically produced by the burning of fossil fuels by the generator of the electricity.

#### Other indirect emissions (scope 3)

Scope 3 emissions are indirect emissions, other than electricity (scope 2), which occur outside of the boundary of an organisation as a result of actions by the organisation. Scope 3 emissions may occur:

upstream, such as the emissions generated in the extraction and production of fossil fuels;

downstream, such as the emissions from transport of an organisation’s product to customers, or the emissions from outsourced activities.

The majority of a company’s value chain greenhouse gas emissions may lie outside their own operations. Emissions from a company’s value chain occurring externally to their operations within Australia may be estimated using the scope 3 emission factors published in this workbook. If you need to calculate emissions occurring outside of Australia, please check relevant resources, such as the IPCC Emission Factor database for more appropriate location specific emission factors.

### Using emission factors

Emission factors are used to convert a unit of activity into its emissions equivalent. Greenhouse gas emissions are calculated by multiplying the relevant source specific emission factor, by the quantity of the activity, to give the emissions of different greenhouse gases for each source type.

Example: Greenhouse Gas Emissions (t CO2-e) =

Activity Data (GJ) x Emission Factor (t CO2-e /GJ)

When an emission factor is given as t CO2-e /GJ but the activity data is not in GJ, then an energy content factor is also included in the calculation. The energy content factor is the amount of energy contained in fuel, measured in gross calorific value.

Example: Greenhouse Gas Emissions (t CO2-e) =

Activity Data (kL) x Energy Content Factor (GJ/kL) x Emission Factor (t CO2-e /GJ)

It is important to note that an emission factor is activity-specific and that the type of activity determines the emission factor used. Emission factors are generally expressed on a carbon dioxide equivalent (CO2-e) basis using the Global Warming Potential (GWP) factors from the IPCC 5th Assessment report and provided in Appendix 2 Global Warming Potentials. As greenhouse gases vary in their radiative forcing and in their atmospheric residence time, converting emissions into a carbon dioxide equivalent over a 100-year horizon allows the integrated effect of emissions of the various gases to be compared on an equivalent basis.

### Emission factor timing

This issue of the NGA Factors coincides with the release of the NGER (Measurement) Amendment (2024 Update) Determination 2024, issued for the 2024-25 NGER reporting cycle. Data used for the estimation of the factors within this document are the latest available at the time of estimation - which includes NGER reporting data for the 2022-23 cycle.

## Revisions to previous issue

### Updated emission factors

The emission factors reported in this publication replace those listed in the 2023 NGA Factors Workbook.

#### Scope 2 and 3 electricity

The 2024 update to the scope 2 emission factors are available in Table 1 and further details are provided in Appendix 4 Methodology for calculating electricity emission factors.

The Jurisdictional Residual Power Percentage value for the ACT has been updated to include changes in the way it is calculated. It now includes the generation of solar power from small rooftop systems below 100 kW as contributing to the ACT’s renewable power consumption.

#### Waste

Additional information provided in Appendix 5 in relation to the calculation of emission factors for Scope 3 emissions from solid waste disposal.

Updated emission factor for the incineration of municipal solid waste.

# Energy

Emissions in the Energy sector (IPCC Sector 1) arise from fuel combustion, fugitive emissions, and carbon dioxide transport and storage. Fuel combustion includes:

Electricity - Emissions from the combustion of fuels to generate electricity.

Stationary energy - Emissions from the combustion of fuels to generate steam, heat or pressure, other than for electricity generation and transport.

Transport - Emissions from the combustion of fuels for transportation within Australia.

Fugitive emissions are emissions released during the extraction, processing and delivery of fossil fuels. Carbon dioxide transport and storage emissions are any leakage or venting of carbon dioxide during the transport, injection and geological storage of carbon dioxide associated with carbon capture and storage activities.

## Greenhouse gas emissions from energy

The principal greenhouse gas generated by the combustion of fossil fuels for energy is carbon dioxide (CO2). The quantity of gas produced depends on the carbon content of the fuel and the degree to which the fuel is fully combusted.  Additionally, small quantities of methane (CH4) and nitrous oxide (N2O) are also produced, depending on the actual combustion conditions. For example, CH4 may be generated when fuel is only partially combusted. Similarly, N2O results from the reaction between nitrogen and oxygen in the combustion air.

## Estimating emissions from stationary energy sources

### Electricity

Scope 2 emissions are physically produced by the burning of fuels (coal, natural gas, etc.) at the power station to create electricity. Emissions from upstream (e.g. coal transport) and downstream (e.g. transmission and distribution losses) in the electricity supply chain are captured in scope 3 emissions.

- For electricity, the scope 3 emission factor depends on the amount of electricity lost throughout the grid network. These losses depend on:

the distance of the generator from customers - more power is lost the further it travels;

the voltage and resistance of the transmission lines - the “quality” of the line;

- how much power is flowing through the line - a more heavily loaded line means more heat and more losses.

There are two methods for estimating electricity emissions – the location-based method and the market-based method. Reporting electricity emissions under both methods provides different perspectives of the emissions associated with a company’s electricity usage.

The location-based method shows a company’s electricity emissions in the context of its location. It calculates the emissions from a company’s electricity consumption, reflecting the emissions intensity of electricity generation within the state or territory where it operates.

The location-based scope 2 emission factors are state-based emission factors from on-grid electricity generation, calculated from the physical characteristics of the electricity grid. The emission factors are calculated each financial year based on electricity generation within each state and territory and take into account interstate electricity flows, where they exist, and the emissions attributable to those flows. The state-based emission factor calculates an average emission factor for all electricity consumed from the grid in a given state, territory or electricity grid.

Table 1 Indirect (scope 2 and scope 3) emission factors from consumption of purchased or acquired electricity

| State, Territory or grid description | Scope 2 Emission Factors (kg CO2-e/kWh) | Scope 3 Emission Factors (kg CO2-e/kWh) |
| --- | --- | --- |
| New South Wales and Australian Capital Territory | 0.66 | 0.04 |
| Victoria | 0.77 | 0.09 |
| Queensland | 0.71 | 0.10 |
| South Australia | 0.23 | 0.05 |
| Western Australia - South West Interconnected System (SWIS) | 0.51 | 0.06 |
| Western Australia - North Western Interconnected System (NWIS) | 0.61 | 0.09 |
| Tasmania | 0.15 | 0.03 |
| Northern territory - Darwin Katherine Interconnected System (DKIS) | 0.56 | 0.07 |
| National | 0.63 | 0.07 |

Sources: Primary data sources comprise National Greenhouse and Energy Reporting (Measurement) Determination 2008 (Schedule 1), Australian Energy Statistics, Clean Energy Regulator, and AEMO data and Department of Climate Change, Energy, the Environment and Water.

Notes:

Data are for financial years ending in June.

These factors include methane emissions from hydro dams. This impacts Tasmania, NSW/ACT and Victoria.

Self-consumption of small-scale rooftop solar is deducted from the estimation of these factors.

To convert to kg CO2-e/GJ, multiply by 1000 then divide by 3.6.

Depending on the intended use, the publication of these revised factors does not necessarily imply any need to revise past estimates of emissions. Historic emission factor estimates are available in previous releases of the NGA Factors.

For further information on the estimation of scope 2 emissions, see Appendix 4 Methodology for calculating electricity

- The market-based method shows a company’s electricity emissions in the context of its investments in different electricity products and markets. This includes from voluntary purchases of renewable electricity and mandatory schemes like the Large-scale Renewable Energy Target.

The market-based method assigns an emissions factor of zero for a company’s investments in renewable electricity and uses a national residual mix factor to calculate emissions from any remaining electricity consumption.

- Companies should be aware that when they are consuming electricity from the grid, they are consuming electricity from a range of generating technologies in operation at the time of consumption, which may include non-renewable sources of generation. The market-based method allows companies to match their consumption with investments in renewable electricity, but it does not mean that they are only consuming electricity from renewable generators.

Table 2 Indirect (scope 2 and scope 3) emission factors from consumption of purchased or acquired electricity: market-based factors

| Residual Mix Factor for the market-based method | Scope 2 Emission Factors (kg CO2-e/kWh) | Scope 3 Emission Factors (kg CO2-e/kWh) |
| --- | --- | --- |
| National | 0.81 | 0.11 |

Sources: Primary data sources comprise National Greenhouse and Energy Reporting (Measurement) Determination 2008 (Schedule 1), Australian Energy Statistics, Clean Energy Regulator, and AEMO data and Department of Climate Change, Energy, the Environment and Water.

Notes:

An adjustment has been made to include methane emissions from hydro dams.

Self-consumption of small-scale rooftop solar is deducted from the estimation of these factors.

To convert to kg CO2-e/GJ, multiply by 3.6 then divide by 1000.

Depending on the intended use, the publication of these revised factors does not necessarily imply any need to revise past estimates of emissions. Historic emission factor estimates are available in previous NGA Factors Workbooks.

For further information on the estimation of scope 2 emissions, see Appendix 4 Methodology for calculating electricity

#### Calculating emissions from electricity purchased or acquired using the location-based method

The following method is used for estimating scope 2 and scope 3 emissions released from electricity purchased or acquired and consumed using the location-based method:

t CO2˗e = Q  ×EF2+ EF31,000

Where:

t CO2-e is the emissions measured in CO2-e tonnes.

Q is the quantity of electricity purchased or acquired and consumed from the operation of the facility during the year measured in kilowatt hours.

EF2 is the scope 2 emission factor, in kilograms of CO2-e emissions per kilowatt hour, as per Table 1.

EF3 is the scope 3 emission factor, in kilograms of CO2-e emissions per kilowatt hour, as per Table 1.

Note: As the emission factors are given in kg CO2-e, the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 1 Calculation of scope 2 and 3 emissions (t CO2-e) from purchased electricity (kWh) using the location-based method

A company has operations in New South Wales and Victoria. A component of the company’s energy use is electricity purchased from the National Electricity Market grid. During the year the NSW operations consumed 11,300,000 kWh of electricity, while the Victoria operations consumed 14,600,000 kWh. Emissions are estimated using the equation below:

t CO2˗e= Q ×EF2+EF31,000

Where: NSW Q = 11,300,000 kWh EF2 = 0.66 kg CO2-e /kWh EF3 = 0.04 kg CO2-e /kWh Victoria Q = 14,600,000 kWh EF2 = 0.77 kg CO2-e /kWh EF3 = 0.09 kg CO2-e /kWh

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

For NSW operations =11,300,000 ×(0.66+0.04)1000 = 7,910 t CO2-e

For Victoria operations =14,600,000 ×0.77+0.091000 = 12,556 t CO2-e

Combined total greenhouse gas emissions (t CO2-e) = 20,466 t CO2-e

#### Calculating emissions from electricity purchased or acquired using the market-based method

The following method may be used for estimating scope 2 and scope 3 emissions released from electricity purchased or acquired and consumed using the market-based method:

##### t CO2˗e = Q-Qexempt×1-RPP+JRPP+Qexempt× 1-JRPP-RECsurr-REConsite×1,000×RMF1+RMF21,000

Where:

t CO2-e is the emissions measured in CO2-e tonnes.

Q is the quantity of electricity purchased or acquired and consumed from the operation of the facility during the year measured in kilowatt hours.

Qexempt is the quantity of electricity exempt from Renewable Energy Target (RET) liability, measured in kilowatt hours.

RPP is the RET Renewable Power Percentage for the applicable period as published by the Clean Energy Regulator, averaged across the previous and current calendar years. For example, calendar years 2023 and 2024 are used for the calculation of the financial year 2024 RPP.

JRPP is the jurisdictional renewable power percentage for the applicable period and activity state. It is calculated as the number of eligible Renewable Energy Certificates surrendered by or on behalf of the jurisdictional authority divided by total electricity consumption in the jurisdiction.

RECsurr is the number of eligible Renewable Energy Certificates voluntarily surrendered in the reporting year equivalent to megawatt hours.

REConsite is the number of eligible Renewable Energy Certificates that have been or will be issued for electricity produced on-site during the year and consumed from the operation of the facility equivalent to megawatt hours.

RMF1 is the scope 2 residual mix factor, in kilograms of CO2-e emissions per kilowatt hour, as per Table 2.

RMF2 is the scope 3 residual mix factor, in kilograms of CO2-e emissions per kilowatt hour, as per Table 2.

Notes:

As the emission factors are given in kg CO2-e, it is necessary to divide by 1,000 when reporting in t CO2-e.

The ACT is currently the only state with a JRPP. The JRPP for the ACT for the 2024 reporting period is 79.51%.

The RET Renewable Power Percentage is published by the Clean Energy Regulator.

An eligible Renewable Energy Certificate is:

  - a Large-scale Generation Certificate (LGC) that is voluntarily surrendered through the Renewable Energy Certificate Registry in the reporting year with a generation date of less than 36 months prior to the end of the reporting year; or

  - a purchase of GreenPower electricity from an accredited GreenPower Provider.

##### Example 2 Calculation of scope 2 and 3 emissions (t CO2-e) from purchased or acquired electricity (kWh) using the market-based method

A company has operations in New South Wales. A component of the company’s energy use is electricity purchased from the National Electricity Market grid. During the year the NSW operations consumed 11,300,000 kWh of electricity from the grid. The company generated and consumed 500,000 kWh of electricity from onsite renewable electricity sources, for which LGCs were created. 200,000 kWh of these LGCs were surrendered to meet the company’s RET liability, while the remaining 300,000 kWh were voluntarily surrendered. The company’s activities are not in a jurisdiction whose jurisdictional authority voluntarily surrenders Renewable Energy Certificates. The company purchased and voluntarily surrendered 1,000 LGCs in the same period. Emissions are estimated using the equation below:

t CO2˗e = Q-Qexempt×1-RPP+JRPP+Qexempt× 1-JRPP-RECsurr-REConsite×1,000×RMF1+RMF21,000

Where: Q = 11,300,000 kWh Qexempt = 0.0 RPP = 0.1872 (value for average of published RPP for 2023 and 2024 calendar years) JRPP = 0.0 RECsurr = 1,000 + 300 MWh REConsite = 500 MWh of LGC creations RMF1 = 0.81 kg CO2-e/kWh RMF2 = 0.11 kg CO2-e/kWh

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

=11,300,000-0.0×1-0.1872+0.0+0.0× 1-0.0-1,300-500×1,000×0.81+0.111,000

Combined total greenhouse gas emissions (t CO2-e) = 7,254 t CO2-e

### Emissions factors for the Carbon Credits (Carbon Farming Initiative) Act 2011

This section contains emissions factors for electricity grids to be used for calculating a carbon dioxide equivalent net abatement amount in accordance with a methodology determination under the Carbon Credits (Carbon Farming Initiative) Act 2011. These emission factors are listed in Table 3.

Table 3: Indirect (scope 2) emission factors for electricity grids

| Electricity grid | Emission factor (kg CO2-e/kWh) |
| --- | --- |
| National Electricity Market (NEM) | 0.64 |
| Western Australia - South West Interconnected System (SWIS) | 0.51 |
| Northern territory - Darwin Katherine Interconnected System (DKIS) | 0.56 |
| Western Australia - North Western Interconnected System (NWIS) | 0.61 |
| Off-grid | 0.67 |

Sources: National Greenhouse and Energy Reporting (Measurement) Determination 2008 
(Schedule 1) and Department of Industry, Science, and Resources, Australian Energy Statistics, Clean Energy Regulator, and AEMO data and Department of Climate Change, Energy, the Environment and Water.

### Stationary combustion of solid fuels

Table 4 Direct (scope 1) and Indirect (scope 3) emission factors for the combustion of solid fuels - including certain coal based products

| Fuel combusted | Energy Content factor (GJ/t) | Scope 1 Emission Factor (kg CO2-e/GJ) CO2 | Scope 1 Emission Factor (kg CO2-e/GJ) CH4 | Scope 1 Emission Factor (kg CO2-e/GJ) N2O | Scope 1 Emission Factor (kg CO2-e/GJ) Combined gases | Scope 3 Emission Factor (kg CO2-e /GJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Bituminous coal | 27.0 | 90 | 0.04 | 0.2 | 90.24 | 3.0 |
| Sub-bituminous coal | 21.0 | 90 | 0.04 | 0.2 | 90.24 | 2.5 |
| Anthracite | 29.0 | 90 | 0.04 | 0.2 | 90.24 | NE |
| Brown coal (lignite) | 10.2 | 93.5 | 0.02 | 0.3 | 93.82 | 0.4 |
| Coking coal | 30.0 | 91.8 | 0.03 | 0.2 | 92.03 | 6.4 |
| Coal briquettes | 22.1 | 95 | 0.08 | 0.3 | 95.38 | NE |
| Coal coke | 27.0 | 107 | 0.03 | 0.2 | 107.23 | NE |
| Coal tar | 37.5 | 81.8 | 0.03 | 0.2 | 82.03 | NE |
| Solid fossil fuels other than those mentioned in the items above | 22.1 | 95 | 0.08 | 0.2 | 95.28 | NE |
| Industrial materials that are derived from fossil fuels, if recycled and combusted to produce heat or electricity | 26.3 | 81.6 | 0.03 | 0.2 | 81.83 | NE |
| Passenger car tyres, if recycled and combusted to produce heat or electricity | 32.0 | 62.8 | 0.03 | 0.2 | 63.03 | NE |
| Truck and off-road tyres, if recycled and combusted to produce heat or electricity | 27.1 | 55.9 | 0.03 | 0.2 | 56.13 | NE |
| Non-biomass municipal materials, if combusted to produce heat or electricity | 10.5 | 87.1 | 0.8 | 1.0 | 88.9 | NE |
| Dry wood | 16.2 | 0 | 0.1 | 1.1 | 1.2 | NE |
| Green and air dried wood | 10.4 | 0 | 0.1 | 1.1 | 1.2 | NE |
| Sulphite lyes | 12.4 | 0 | 0.08 | 0.5 | 0.58 | NE |
| Bagasse | 9.6 | 0 | 0.3 | 1.1 | 1.4 | NE |
| Biomass, municipal and industrial materials, if combusted to produce heat or electricity | 12.2 | 0 | 0.8 | 1.0 | 1.8 | NE |
| Charcoal | 31.1 | 0 | 5.3 | 1.0 | 6.3 | NE |
| Primary solid biomass fuels other than those mentioned in the items above | 12.2 | 0 | 0.8 | 1.0 | 1.8 | NE |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008 (Schedule 1) and Department of Climate Change, Energy, the Environment and Water.

Notes:

All emission factors incorporate relevant oxidation factors (sourced from the National Inventory Report).

All emission factors expressed in terms of energy measured as gross calorific value (GCV) equivalents.

Energy content and emission factors for coal products are measured on an as combusted basis.  The energy content for black coal types and coking coal (metallurgical coal) is on a washed basis.

Biofuels CO2 emission factors assume emissions and removals due to the harvesting and regrowth of biomass are reported in the relevant land use category of the Land sectors where the biomass originates and are in balance.

NE = not estimated. No data available to support the provision for a scope 3 factor of these fuels.

#### Calculating emissions from stationary combustion of solid fuels

The following formula can be used to estimate greenhouse gas emissions from the combustion of each type of fuel listed in Table 4.

t CO2˗e=Q × EC ×EF1+EF31,000

Where:

t CO2-e is the emissions of gas type measured in CO2-e tonnes.

Q is the quantity of fuel type measured in tonnes or gigajoules.

EC is the energy content factor of the fuel (gigajoules per tonne) according to each fuel in Table 4.

EF1 is the scope 1 emission factor, in kilograms of CO2-e per gigajoule, for each gas type and for each fuel type as per Table 4.  These can also be added together to get a combined emission factor, to ensure CO2, CH4 and N2O emissions are included, as per Example 2.

EF3 is the scope 3 emission factor, in kilograms of CO2-e per gigajoule, for each gas type and for each fuel type as per Table 4.

Notes:

If Q is specified in gigajoules, then EC is 1.

As the emission factors are given in kg CO2-e, the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 3 Calculation of emissions (t CO2-e) from stationary combustion of 
solid fuel (t)

A facility consumes 20,000 tonnes of brown coal for a purpose other than for the production of electricity or to produce coke. Scope 1 and scope 3 emissions are estimated as follows:

t CO2˗e=Q × EC ×EF1+EF31,000

Where: Q = 20,000 t EC = 10.2 GJ/t EF1 is the Scope 1 emission factor for each individual gas as per Table 4, which can be combined into a single emission factor. carbon dioxide CO2 = 93.50 kg CO2-e/GJ methane CH4 = 0.02 kg CO2-e/GJ nitrous oxide N2O = 0.30 kg CO2-e/GJ EF3 from Table 4 = 0.40 kg CO2-e/GJ

Solution 1: Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Combined emission factor =93.5+0.02+0.30+0.40 = 94.22 kg CO2-e/GJ

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) =20,000 × 10.2 × 94.221,000= 19,221 t CO2-e

Solution 2: Calculation of Emissions of Individual Greenhouse Gases (t CO2-e)

Scope 1 Emissions of carbon dioxide =20,000 × 10.2 × 93.51,000= 19,074 t CO2-e

Scope 1 Emissions of methane =20,000 × 10.2 × 0.021,000= 4 t CO2-e

Scope 1 Emissions of nitrous oxide =20,000 × 10.2 × 0.301,000= 61 t CO2-e

Scope 3 Emissions =20,000 × 10.2 × 0.401,000= 82 t CO2-e

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) = 19,221 t CO2-e

### Stationary combustion of gaseous fuels

Table 5 Direct (Scope 1) emission factors for the consumption of gaseous fuels including liquefied natural gas

| Fuel combusted | Energy Content factor (GJ/m3 unless otherwise indicated) | Scope 1 Emission Factor (kg CO2-e/GJ) CO2 | Scope 1 Emission Factor (kg CO2-e/GJ) CH4 | Scope 1 Emission Factor (kg CO2-e/GJ) N2O | Scope 1 Emission Factor (kg CO2-e/GJ) Combined gases |
| --- | --- | --- | --- | --- | --- |
| Natural gas distributed in a pipeline | 0.0393 | 51.4 | 0.1 | 0.03 | 51.53 |
| Coal seam methane that is captured for combustion | 0.0377 | 51.4 | 0.2 | 0.03 | 51.63 |
| Coal mine waste gas that is captured for combustion | 0.0377 | 51.9 | 4.6 | 0.3 | 56.8 |
| Compressed natural gas (reverting to standard conditions) | 0.0393 | 51.4 | 0.1 | 0.03 | 51.53 |
| Unprocessed natural gas | 0.0393 | 51.4 | 0.1 | 0.03 | 51.53 |
| Ethane | 0.0629 | 56.5 | 0.03 | 0.03 | 56.56 |
| Coke oven gas | 0.0181 | 37.0 | 0.03 | 0.05 | 37.08 |
| Blast furnace gas | 0.0040 | 234.0 | 0.03 | 0.02 | 234.05 |
| Town gas | 0.0390 | 60.2 | 0.04 | 0.03 | 60.27 |
| Liquefied natural gas | 25.3 GJ/kL | 51.4 | 0.1 | 0.03 | 51.53 |
| Gaseous fossil fuels other than those mentioned in the items above | 0.0390 | 51.4 | 0.1 | 0.03 | 51.53 |
| Landfill biogas that is captured for combustion (methane only) | 0.0377 | 0.0 | 6.4 | 0.03 | 6.43 |
| Sludge biogas that is captured for combustion (methane only) | 0.0377 | 0.0 | 6.4 | 0.03 | 6.43 |
| A biogas that is captured for combustion, other than those mentioned in the items above | 0.0370 | 0.0 | 6.4 | 0.03 | 6.43 |
| Biomethane | 0.0393 | 0.0 | 0.1 | 0.03 | 0.13 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008 
(Schedule 1)

Notes:

All emission factors incorporate relevant oxidation factors (sourced from the National Inventory Report).

All EFs expressed in terms of energy measured as gross calorific value (GCV) equivalents Biofuels CO2 emission factors assume emissions and removals due to the harvesting and regrowth of biomass are reported in the relevant land use category of the Land sectors where the biomass originates and are in balance.

No data available to calculate scope 3 for gaseous fuels other than natural gas (Table 6) and ethane 
(Table 7).

Energy content is measured on a dry gas bases under standard conditions:

  - air pressure of 101.325 kilopascals;

  - air temperature of 15.0 degrees Celsius; and

  - air density of 1.225 kilograms per cubic metre.

Table 6 Indirect (Scope 3) emission factors for the consumption of natural gas

| State or territory | Scope 3 Emission Factors for Natural Gas (kg CO2-e/GJ) Metro | Scope 3 Emission Factors for Natural Gas (kg CO2-e/GJ) Non-Metro |
| --- | --- | --- |
| New South Wales and ACT | 13.1 | 14.0 |
| Victoria | 4.0 | 4.0 |
| Queensland | 8.8 | 7.9 |
| South Australia | 10.7 | 10.6 |
| Western Australia | 4.1 | 4.0 |
| Tasmania | C | C |
| Northern Territory | C | C |

Source: Wilkenfeld and Associates (2012), derived from NGER data.

Notes:

As Scope 3 factors are calculated at the point where natural gas is delivered to end users, it is necessary to calculate consumption at each of the load centres. Gas reaches end users either from the metro distribution networks or direct from the high-pressure distribution pipelines. Households and commercial users tend to be served by the former, and electricity generators and large industrial plants from the latter.

Metro is defined as located on or east of the dividing range in NSW, including Canberra and Queanbeyan, Melbourne, Brisbane, Adelaide or Perth. Otherwise, the non-metro factor should be used.

Factors are calculated for all natural gas distributed in a pipeline which may include coal seam methane.

Table 6 is not applicable to ethane. See Table 7 Indirect (Scope 3) emissions from the consumption of ethane.

C = Confidential. Scope 3 emission factors for Tasmania and the Northern Territory are not available due to confidentiality constraints that arise from the use of a limited number of NGER data inputs. It is suggested that for Tasmania the use of the Victorian emission factors is appropriate, while for Northern Territory the use of the Western Australian emission factors is appropriate.

Scope 3 emission factors do not include fugitive emission leakage from low pressure natural gas distribution pipeline networks while those from high pressure gas networks are considered negligible.

Table 7 Indirect (Scope 3) emission factors for the consumption of ethane

| Ethane produced in accordance with Division 23 of Part 3 of the Clean Energy Regulations 2011 | Scope 3 Emission Factors for Ethane (kg CO2-e /GJ) |
| --- | --- |
| New South Wales | 23.7 |
| Victoria | 5.7 |

Source: Wilkenfeld and Associates (2012), derived from NGER data

#### Calculating emissions from stationary combustion of gaseous fuels

The following formula can be used to estimate greenhouse gas emissions from the combustion of each type of fuel listed in Table 5.

t CO2˗e =Q × EC × (EF1 + EF3)1,000

Where:

t CO2-e is the emissions of each gas type, from each gaseous fuel type, measured in CO2-e tonnes.

Q is the quantity of fuel type (e.g. cubic metres, gigajoules or kilolitres)

EC is the energy content factor of fuel type (gigajoules per cubic metre), according to Table 5.

EF1 is the scope 1 emission factor, in kilograms of CO2-e per gigajoule, for each gas type and for each fuel type as per Table 4.  These can also be added together to get a combined emission factor, to ensure CO2, CH4 and N2O emissions are included, as per Example 4.

EF3 is the scope 3 emission factor, in kilograms of CO2-e per gigajoule, as per Table 6.

Notes:

If Q is specified in gigajoules, then EC is 1.

As the emission factors are given in kg CO2-e, the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 4 Calculation of emissions (t CO2-e) from natural gas consumption (GJ)

A facility, in Sydney NSW, consumes 100,000 gigajoules of natural gas (distributed in a pipeline). Scope 1 and scope 3 emissions are estimated as follows:

t CO2˗e =Q × EC ×EF1+ EF31,000

Where: Q = 100,000 GJ EC = 1 GJ/GJ EF1 is the Scope 1 emission factor for each individual gas as per Table 5, which can be combined into a single emission factor. carbon dioxide CO2 = 51.4 kg CO2-e/GJ methane CH4 = 0.1 kg CO2-e/GJ nitrous oxide N2O = 0.03 kg CO2-e/GJ EF3 as per Table 6 = 13.1 kg CO2-e/GJ

Solution 1: Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Combined emission factor =51.4 + 0.1 + 0.03 +13.1 = 64.63

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) =100,000 × 1 × 64.631,000 = 6,463 t CO2-e

Solution 2: Calculation of Emissions of Individual Greenhouse Gases (t CO2-e)

Scope 1 Emissions of carbon dioxide =100,000 × 1 × 51.41,000 = 5,140 t CO2-e

Scope 1 Emissions of methane =100,000 × 1 × 0.11,000 = 10 t CO2-e

Scope 1 Emissions of nitrous oxide =100,000 × 1 × 0.031,000 = 3 t CO2-e

Scope 3 Emissions =100,000 × 1 × 13.11,000 = 1,310 t CO2-e

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) = 6,463 t CO2-e

##### Example 5 Calculation of emissions (t CO2-e) from liquefied natural gas 
consumption (kL)

A facility consumes 1150 kilolitres (kL) of liquefied natural gas. As there is no scope 3 emission factor provided for liquefied natural gas, the scope 1 emissions are estimated as follows:

t CO2˗e =Q × EC × EF1,000

Where: Q = 1150 kL EC = 25.3 GJ/kL EF1 is the Scope 1 emission factor for each individual gas as per Table 5, which can be combined into a single emission factor. carbon dioxide CO2 = 51.4 kg CO2-e/GJ methane CH4 = 0.1 kg CO2-e/GJ nitrous oxide N2O = 0.03 kg CO2-e/GJ

Solution 1: Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Combined emission factor =51.4+ 0.1 + 0.03 = 51.53 kg CO2-e/GJ

Combined total greenhouse gas emissions (t CO2-e) =1150× 25.3 × 51.531,000 = 1,499 t CO2-e

Solution 2: Calculation of Emissions of Individual Greenhouse Gases (t CO2-e)

Scope 1 Emissions of carbon dioxide =1150 × 25.3 × 0.11,000 = 3 t CO2-e

Scope 1 Emissions of methane =1150 × 25.3 × 0.031,000 = 1 t CO2-e

Scope 1 Emissions of nitrous oxide =1150 × 25.3 × 0.031,000 = 1 t CO2-e

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) = 1,499 t CO2-e

### Stationary combustion of liquid fuels

Table 8 Direct (Scope 1) and indirect (scope 3) emission factors for the consumption of liquid fuels, including certain petroleum based products for stationary energy purposes

| Fuel combusted | Energy Content factor (GJ per unit of fuel) | Scope 1 Emission Factor (kg CO2-e/GJ) CO2 | Scope 1 Emission Factor (kg CO2-e/GJ) CH4 | Scope 1 Emission Factor (kg CO2-e/GJ) N2O | Scope 1 Emission Factor (kg CO2-e/GJ) Combined gases | Scope 3 Emission Factor (kg CO2-e /GJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Petroleum based oils (other than petroleum based oil used as fuel), e.g. lubricants | 38.8 GJ/kL | 13.9 | 0.0 | 0.0 | 13.9 | 18.0 |
| Petroleum based greases | 38.8 GJ/kL | 3.5 | 0.0 | 0.0 | 3.5 | 18.0 |
| Crude oil including crude oil condensates | 45.3 GJ/t | 69.6 | 0.08 | 0.2 | 69.88 | NE |
| Other natural gas liquids | 46.5 GJ/t | 61.0 | 0.08 | 0.2 | 61.28 | NE |
| Automotive gasoline/petrol (other than for use as fuel in an aircraft) | 34.2 GJ/kL | 67.4 | 0.2 | 0.2 | 67.80 | 17.2 |
| Aviation gasoline | 33.1 GJ/kL | 67 | 0.2 | 0.2 | 67.40 | 18.0 |
| Kerosene (other than for use as fuel in an aircraft) | 37.5 GJ/kL | 68.9 | 0.01 | 0.2 | 69.11 | 18.0 |
| Aviation turbine fuel/kerosene | 36.8 GJ/kL | 69.6 | 0.02 | 0.2 | 69.82 | 18.0 |
| Heating oil | 37.3 GJ/kL | 69.5 | 0.03 | 0.2 | 69.73 | 18.0 |
| Diesel oil | 38.6 GJ/kL | 69.9 | 0.1 | 0.2 | 70.20 | 17.3 |
| Fuel oil | 39.7 GJ/kL | 73.6 | 0.04 | 0.2 | 73.84 | 18.0 |
| Liquefied aromatic hydrocarbons | 34.4 GJ/kL | 69.7 | 0.03 | 0.2 | 69.93 | 18.0 |
| Solvents: mineral turpentine or white spirits | 34.4 GJ/kL | 69.7 | 0.03 | 0.2 | 69.93 | 18.0 |
| Liquefied petroleum gas (LPG) | 25.7 GJ/kL | 60.2 | 0.2 | 0.2 | 60.60 | 20.2 |
| Naphtha | 31.4 GJ/kL | 69.8 | 0.01 | 0.01 | 69.82 | 18.0 |
| Petroleum coke | 34.2 GJ/t | 92.6 | 0.08 | 0.2 | 92.88 | 18.0 |
| Refinery gas and liquids | 42.9 GJ/t | 54.7 | 0.03 | 0.03 | 54.76 | 18.0 |
| Refinery coke | 34.2 GJ/t | 92.6 | 0.08 | 0.2 | 92.88 | 18.0 |
| Petroleum based products other than mentioned in the items above | 34.4 GJ/kL | 69.8 | 0.02 | 0.1 | 69.92 | 18.0 |
| Biodiesel | 34.6 GJ/kL | 0.0 | 0.08 | 0.2 | 0.28 | NE |
| Ethanol for use as a fuel in an internal combustion engine | 23.4 GJ/kL | 0.0 | 0.08 | 0.2 | 0.28 | NE |
| Biofuels other than those mentioned in the items above and below | 23.4 GJ/kL | 0.0 | 0.08 | 0.2 | 0.28 | NE |
| Renewable aviation kerosene | 36.8 GJ/kL | 0.0 | 0.02 | 0.2 | 0.22 | NE |
| Renewable diesel | 38.6 GJ/kL | 0.0 | 0.1 | 0.2 | 0.30 | NE |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008 (Schedule 1).

Notes:

All emission factors incorporate relevant oxidation factors (sourced from the National Inventory Report).

All EFs are expressed in terms of energy measured as gross calorific value (GCV) equivalents.

Biofuels CO2 emission factors assume emissions and removals due to the harvesting and regrowth of biomass are reported in the relevant land use category of the Land sectors where the biomass originates and are in balance.

Scope 3 factors for biofuels such as biodiesels and ethanol are highly dependent on individual plant and project characteristics, and therefore have not been estimated.

NE = not estimated. No or limited data available to support the provision for a scope 3 factor of these fuels.

#### Calculating emissions from stationary combustion of liquid fuels

The following formula can be used to estimate greenhouse gas emissions from the stationary combustion of each type of liquid fuel listed Table 8.

t CO2˗e=Q × EC ×EF1+EF31,000

Where:

t CO2-e is the emissions of each gas type, from each fuel type, measured in CO2-e tonnes.

Q is the quantity of fuel type measured in kilolitres, tonnes or gigajoules.

EC is the energy content factor of the fuel (gigajoules per unit of fuel type) according to each fuel in Table 8.

EF1 is the scope 1 emission factor, in kilograms of CO2-e per gigajoule, for each gas type and for each fuel type as per Table 6. These can also be added together to get a combined emission factor, to ensure CO2, CH4 and N2O emissions are included, as per Example 6.

EF3 is the scope 3 emission factor, in kilograms of CO2-e per gigajoule, for each gas type and for each fuel type as per Table 8.

Notes:

If Q is specified in gigajoules, then EC is 1.

As the emission factors are given in kg CO2-e, the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 6 Calculation of emissions (t CO2-e) from stationary diesel oil 
consumption (kL)

A facility consumes 700 kilolitres (kL) of diesel oil in their on-site generator. Scope 1 and scope 3 emissions are estimated as follows:

t CO2˗e=Q × EC ×EF1+EF31,000

Where: Q = 700 kL EC = 38.6 GJ/kL EF1 is the Scope 1 emission factor for each individual gas as per Table 8 , which can be combined into a single emission factor. carbon dioxide CO2 = 69.90 kg CO2-e/GJ methane CH4 = 0.1 kg CO2-e/GJ nitrous oxide N2O = 0.2 kg CO2-e/GJ EF3 from Table 8 = 17.3 kg CO2-e/GJ

Solution 1: Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Combined emission factor =69.9 + 0.1 + 0.2 +17.3 = 87.5 kg CO2-e/GJ

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) =700 × 38.6 × 87.5 1,000 = 2,364.3 t CO2-e

Solution 2: Calculation of Emissions of Individual Greenhouse Gases (t CO2-e)

Emissions of carbon dioxide =700 × 38.6 × 69.901,000 = 1,888.7 t CO2-e

Emissions of methane =700 × 38.6 × 0.11,000 = 2.7 t CO2-e

Emissions of nitrous oxide =700 × 38.6 × 0.21,000 = 5.4 t CO2-e

Scope 3 Emissions =700 × 38.6 ×17.31,000 = 467.4 t CO2-e

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) = 2,364.3 t CO2-e

## Estimating emissions from transport fuels

### Transport fuel emissions

Fuels used for transport purposes produce different methane and nitrous oxide emissions than if the same fuels were used for stationary energy purposes. While CO2 emissions are only dependant on the fuel type, CH4 and N2O emissions are dependent on the type of engine technology used as well as the fuel type. Therefore, separate emission factors are provided in Table 9 to apply where fuels are used for general transport purposes as well as specific transport types.

Table 9 Direct (scope 1) and indirect (scope 3) emission factors for the consumption of transport fuels in different transport equipment

| Transport type | Fuel combusted | Energy Content factor (GJ per unit of fuel) | Scope 1 Emission Factor (kg CO2-e/GJ) CO2 | Scope 1 Emission Factor (kg CO2-e/GJ) CH4 | Scope 1 Emission Factor (kg CO2-e/GJ) N2O | Scope 1 Emission Factor (kg CO2-e/GJ) Combined gases | Scope 3 Emission Factor (kg CO2-e /GJ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cars and light commercial vehicles | Gasoline | 34.2 | 67.4 | 0.02* | 0.2* | 67.62 | 17.2 |
| Cars and light commercial vehicles | Diesel oil | 38.6 | 69.9 | 0.01* | 0.5* | 70.41 | 17.3 |
| Cars and light commercial vehicles | Liquefied petroleum gas (LPG) | 26.2 | 60.2 | 0.5* | 0.3* | 61.00 | 20.2 |
| Cars and light commercial vehicles | Fuel oil | 39.7 | 73.6 | 0.08 | 0.5 | 74.18 | 18.0 |
| Cars and light commercial vehicles | Ethanol | 23.4 | 0.0 | 0.2* | 0.2* | 0.4 | NE |
| Cars and light commercial vehicles | Biodiesel | 34.6 | 0.0 | 0.8 | 1.7 | 2.5 | NE |
| Cars and light commercial vehicles | Renewable diesel | 38.6 | 0.0 | 0.01* | 0.5* | 0.51 | NE |
| Cars and light commercial vehicles | Other biofuels | 23.4 | 0.0 | 0.8 | 1.7 | 2.5 | NE |
| Light duty vehicles | Compressed natural gas | 0.0393 GJ/m3 | 51.4 | 7.3 | 0.3 | 59.0 | 18.0 |
| Light duty vehicles | Liquefied natural gas | 25.3 | 51.4 | 7.3 | 0.3 | 59.0 | 18.0 |
| Heavy duty vehicles | Compressed natural gas | 0.0393 GJ/m3 | 51.4 | 2.8 | 0.3 | 54.5 | 18.0 |
| Heavy duty vehicles | Liquefied natural gas | 25.3 | 51.4 | 2.8 | 0.3 | 54.5 | 18.0 |
| Heavy duty vehicles | Diesel oil - Euro iv or higher | 38.6 | 69.9 | 0.07 | 0.4 | 70.37 | 17.3 |
| Heavy duty vehicles | Diesel oil - Euro iii | 38.6 | 69.9 | 0.1 | 0.4 | 70.4 | 17.3 |
| Heavy duty vehicles | Diesel oil - Euro i | 38.6 | 69.9 | 0.2 | 0.4 | 70.5 | 17.3 |
| Heavy duty vehicles | Renewable diesel – Euro iv or higher | 38.6 | 0.0 | 0.07 | 0.4 | 0.47 | NE |
| Heavy duty vehicles | Renewable diesel – Euro iii | 38.6 | 0.0 | 0.1 | 0.4 | 0.5 | NE |
| Heavy duty vehicles | Renewable diesel – Euro i | 38.6 | 0.0 | 0.2 | 0.4 | 0.6 | NE |
| Aviation | Gasoline for use as fuel in an aircraft | 33.1 | 67.0 | 0.06 | 0.6 | 67.66 | 18.0 |
| Aviation | Kerosene for use as fuel in an aircraft | 36.8 | 69.6 | 0.01 | 0.6 | 70.21 | 18.0 |
| Aviation | Renewable aviation kerosene | 36.8 | 0.0 | 0.01 | 0.6 | 0.61 | NE |

Notes:

* For vehicles manufactured prior to 2004, the following scope 1 emission factors (in kg CO2-e/GJ) should be used instead of those presented in Table 9: Gasoline CH4 =0.6, N2O = 1.6, Diesel oil CH4 =0.1, N2O = 0.4, LPG CH4 =0.7, N2O = 0.6, Ethanol CH4= 0.8, N2O = 1.7, Renewable diesel CH4 = 0.1, N2O = 0.4.

NE = not estimated. Scope 3 factors for biofuels such as biodiesels and ethanol are highly dependent on individual plant and project characteristics. No data are available to support the provision for a scope 3 factor of these fuels.

For compressed natural gas, emission factors are for gas that has converted to standard conditions.

#### Calculating emissions from transport fuels

The following formula can be used to estimate greenhouse gas emissions from the combustion of each type of fuel listed in Table 9 used for transport energy purposes.

t CO2˗e=Q × EC ×EF1+EF31,000

Where:

t CO2-e is the emissions from each gas type and each fuel type measured in CO2-e tonnes.

Q is the quantity of fuel type, measured in kilolitres or gigajoules, and combusted for transport energy purposes

EC is the energy content factor of fuel type (gigajoules per kilolitre or per cubic metre) used for transport energy purposes as per Table 9.

EF1 is the scope 1 emission factor, in kilograms of CO2-e per gigajoule, for each transport type and for each fuel type as per Table 9.

EF3 is the scope 3 emission factor, in kilograms of CO2-e per gigajoule, as per Table 9.

Note:

If Q is specified in gigajoules, then EC is 1.

As the emission factors are given in kg CO2-e, the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 7 Calculation of emissions (t CO2-e) from diesel oil consumption (kL) for transport

A freight company consumes 10,000 kL of automotive diesel for transport purposes, in vehicles that were manufactured after 2004. Scope 1 and scope 3 emissions are estimated as follows:

t CO2˗e=Q × EC ×EF1+EF31000

Where: Q = 10,000 kL EC = 38.6 GJ/kL EF1 is the Scope 1 emission factor for each individual gas as per Table 9, which can be combined into a single emission factor. carbon dioxide CO2 = 69.9 kg CO2-e/GJ methane CH4 = 0.1 kg CO2-e/GJ nitrous oxide N2O = 0.4 kg CO2-e/GJ EF3 from Table 9 = 17.3 kg CO2-e/GJ

Solution 1: Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Combined emission factor = 69.9 +0.1 + 0.4+17.3 = 87.61 kg CO2-e/GJ

Combined scopes 1 and 3 total greenhouse gas emissions (t CO2-e) =10,000 × 38.6 × 87.611,000 = 33,817 t CO2-e

Solution 2: Calculation of Emissions of Individual Greenhouse Gases (t CO2-e)

Scope 1 Emissions of carbon dioxide =10,000× 38.6 × 69.901,000 = 26,981 t CO2-e

Scope 1 Emissions of methane =10,000 × 38.6 × 0.11,000 = 4 t CO2-e

Scope 1 Emissions of nitrous oxide =10,000 × 38.6 × 0.401,000 = 154 t CO2-e

Scope 3 Emissions =10,000 × 38.6 × 17.31,000 = 6,677 t CO2-e

Total Greenhouse Gas Emissions (t CO2-e) = 33,817 t CO2-e

## Estimating fugitive emissions

Fugitive emissions involve the release of non-combustion, greenhouse gases arising from the production and delivery of fossil fuels. Fugitive emissions from solid fuels arise from the production of coal, and emissions from decommissioned mines and coal mine waste gas flaring. Fugitive emissions from oil and gas extraction, production and transport involve venting, flaring, leakage, evaporation and storage losses. Fugitive emissions from carbon dioxide transport and storage arise from the transport, injection and storage of carbon dioxide associated with carbon capture and storage activities. For factors and methods used for estimating fugitive emissions, please refer to Chapter 3 of the National Greenhouse and Energy Reporting (Measurement) Determination 2008.

# Industrial processes and product use

Emissions from non-energy related industrial production and processes (IPCC sector 2) includes emissions from hydrofluorocarbons (HFCs), which are used in refrigerants and air conditioning and the use of carbonates. While there are many sectors within industrial processes and product use (IPPU), the most relevant for organisation and individuals preparing a greenhouse gas inventory is likely to be the calculation of refrigerant leakage from refrigeration and air conditioning, and the use of carbonate and soda ash. Methods and factors for estimating greenhouse gas emissions from industrial processes other than those described in this workbook can be found in the National Greenhouse and Energy Reporting (Measurement) Determination 2008.

## Estimating industrial processes emissions

### Refrigerant leakage

Many refrigerants used in refrigeration and air conditioning equipment are also greenhouse gases. Refer to the manufacturer’s product specification to determine the type and the charge of a refrigerant contained in your appliances. Indicative leakage rates and common refrigerant types with their GWPs are given in Table 10 and Table 11 respectively.

Table 10 Indicative leakage rates for common refrigeration and air-conditioning equipment

| Product | Annual leakage rate (%) |
| --- | --- |
| Domestic refrigerators | 1.7 |
| Transport refrigeration | 15.7 |
| Domestic A/C portable | 2.5 |
| Domestic A/C split | 3.5 |
| Domestic A/C packaged | 2.5 |
| Light vehicle A/C | 6.7 |
| Heavy vehicle A/C | 10.8 |

Source: National Inventory Report 2021

Table 11 Global warming potentials of common refrigerants

| Refrigerant | Global Warming Potentials (AR5) |
| --- | --- |
| R22 (HCFC-22) | 1,760 |
| R32 (HFC-32) | 677 |
| R134A (HFC-134A) | 1,300 |
| R410A (HFC blend) | 1,924 |
| R404A (HFC blend) | 3,943 |

Source: Department of Climate Change, Energy, the Environment and Water.

Note: Global warming potential values for other refrigerants and the compositions of blended refrigerants are located in Appendix 2.

#### Calculating emissions from refrigerant leakage

The following formula should be used to estimate the CO2-e emissions from refrigeration and air-conditioning equipment:

t CO2˗e= GWP ×charge ×leakage rate 1,000

Where:

t CO2-e is the emissions measured in CO2-e tonnes.

GWP is the global warming potential of the gas (see Appendix 2 for a list of GWPs).

Charge is the amount of refrigerant gas contained in the appliance (usually found on the appliance).

Leakage rate is the percentage of total charge leaked from the appliance each year.

Note: As the emission factors are given in kg CO2-e the division by 1,000 is necessary when reporting in t CO2-e.

##### Example 8 Calculation of emissions from refrigerant leakage

A household operates a split system air conditioning unit pre-charged with 3kg of the refrigerant R410A. The emissions are estimated as follows:

t CO2˗e= GWP ×charge ×leakage rate 1,000

Where: GWP = 1,924 Charge = 3.0 kg Leakage rate = 3.5%

Calculation of annual greenhouse gas emissions (t CO2-e)

Total Greenhouse Gas Emissions (t CO2-e) =1924×3 ×0.035 1000 = 0.2020 t CO2-e

### Use of carbonates for the production of a product (other than cement clinker, lime or soda ash)

This section applies to calcination or any other use of carbonates that generates CO2 (excluding cement clinker, lime production or soda ash production) including the in-house lime production in the ferrous metals industry. Other examples of industrial processes involving the consumption of carbonates can be found in the National Greenhouse and Energy Reporting (Measurement) Determination 2008.

Table 12 Direct (scope 1) emission factors for the calcination of carbonates

| Source of carbonate consumption | Emission factor (t CO2/tonne) |
| --- | --- |
| Limestone (calcium carbonate) | 0.440 |
| Magnesium carbonate | 0.522 |
| Dolomite | 0.477 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 4.8, Chapter 4, Part 4.2, Division 4.2.1.

#### Calculating emissions from carbonate use

The method for estimating emissions from the use of carbonates for the production of a product other than cement clinker, lime or soda ash is described below:

t CO2˗e = Q × EF × Fcal

Where:

t CO2-e the annual emissions of CO2 from the consumption of carbonate measured in CO2-e tonnes.

Q is the quantity of raw carbonate material consumed measured in tonnes.

EF is the CO2 emission factor for each carbonate material type (tonnes CO2 per tonne of carbonate) as per Table 12.

Fcal is the fraction of the carbonate calcined.  If the information is not available the degree is assumed to be 100%, that is, Fcal =1.

### Use of carbonates in clay materials

Process-related emissions from ceramics result from the calcination of carbonates in the clay, as well as the addition of additives. Ceramics include the production of bricks and roof tiles, vitrified clay pipes, refractory products, expanded clay products, wall and floor tiles, table and ornamental ware (household ceramics), sanitary ware, technical ceramics, and inorganic bonded abrasives. Default scope 1 emission factors are provided by State and Territory.

Table 13 Direct (scope 1) emission factors for use of carbonates in clay materials by state and territory

| State or Territory | Inorganic carbon content factor | Scope 1 emission factor t CO2-e/t clay material |
| --- | --- | --- |
| New South Wales | 0.0061 | 0.0222 |
| Victoria | 0.0002 | 0.0009 |
| Queensland | 0.0025 | 0.0092 |
| Western Australia | 0.0003 | 0.0012 |
| South Australia | 0.0005 | 0.0019 |
| Tasmania | 0.0011 | 0.0038 |
| Australian Capital Territory | 0.0061 | 0.0222 |
| Northern Territory | 0.0005 | 0.0019 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 4.23, Chapter 4, Part 4.2, Division 4.2.3.

#### Calculating emissions from use of carbonates in clay materials

Calculate the emissions of carbon dioxide released from each clay material consumed in the industrial process during the reporting year, measured in CO2-e tonnes, using the following formula:

t CO2˗e =  Q × EF

Where:

t CO2-e is the emissions of carbon dioxide released from the clay material consumed in the industrial process during the reporting year measured in CO2-e tonnes.

Q is the quantity of clay material consumed in the industrial process during the reporting year in a State or Territory measured in tonnes

EF is the scope 1 emission factor, in tonnes of CO2-e per tonnes of clay material as per Table 13.

### Soda ash use

Soda ash is used in a variety of applications, including glass production, soaps and detergents, flue gas desulphurisation, chemicals, pulp and paper and other common consumer products. Soda ash production and consumption (including sodium carbonate, Na2CO3) results in the release of CO2.

Table 14 Direct (scope 1) emission factors for the consumption of soda ash

| Source | Scope 1 emission factor (t CO2-e/t) |
| --- | --- |
| Soda Ash Use | 0.415 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 4.29, Chapter 4, Part 4.2, Division 4.2.4.

#### Calculating emissions from soda ash consumption

This method is derived from those used in the National Greenhouse Accounts. It involves the multiplication of the quantity of soda ash consumed in the production process by the emission factor for soda ash.

t CO2˗e= Q × EF

Where:

t CO2-e the emissions of CO2 from the use of soda ash (CO2-e tonnes)

Q is the amount of soda ash consumed measured in tonnes

EF is the CO2 emission factor (tonnes of CO2 / tonnes of soda ash used) as per Table 14.

# Waste emissions

Emissions from the waste sector (IPCC Sector 5) comprise emissions from the following categories:

- Solid waste disposal

- Wastewater treatment and discharge

- Biological treatment of solid waste

- Incineration and open burning of waste.

Typically, emissions from solid waste disposal are the largest source of greenhouse gas emissions in the waste sector, followed by wastewater treatment and discharge. Emissions from incineration of waste and biological treatment of solid waste are also considered within this chapter.

## Greenhouse gas emissions from waste

Emissions from the waste sector are predominantly CH4, however small amounts of CO2 and N2O are also generated. Emissions of CO2 generated from solid waste disposal and wastewater treatment and discharge are considered to be of biogenic origin (e.g. paper, wood) and do not need to be estimated. For example, when landfills recover methane for flaring or combustion, the CO2 emitted is not considered because it is seen as part of the natural carbon cycle.

## Estimating solid waste emissions

For companies and individuals wishing to calculate the indirect emissions (scope 3) from the disposal of their waste outside the organisation boundaries, such as waste taken to municipal landfill, please use Table 15 and Table 16 below.

Table 15 includes emission factors to be used for various waste types when the weight of waste is known. If waste is measured by volume and not by weight, conversion factors are also in 4.

For those who do not know the composition of their waste can use the emission factors in Table 16, which gives the weighted average emission factors for the municipal, commercial and industrial, and construction and demolition waste categories.

For companies that operate landfill sites and wish to calculate scope 1 emissions, please refer to Chapter 5 – Part 5.2 of the National Greenhouse and Energy Reporting (Measurement) Determination 2008 for guidance.

### Emissions from solid waste disposal to landfill

Table 15 Waste mix methane conversion factors and emission factors

| Waste types | Scope 3 emission factor (t CO2-e/t) | Volume to mass conversion factor (t/m3) |
| --- | --- | --- |
| Food | 2.1 | 0.50 |
| Paper and cardboard | 3.3 | 0.09 |
| Garden and green | 1.6 | 0.24 |
| Wood | 0.7 | 0.15 |
| Textiles | 2.0 | 0.14 |
| Sludge | 0.4 | 0.72 |
| Nappies | 2.0 | 0.39 |
| Rubber and leather | 3.3 | 0.14 |
| Inert waste (including concrete/metal/plastics/glass) | - | 0.42 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008.

Notes:

This method is used to produce an estimate of lifetime emissions from waste degradation in a landfill.  In reality, waste disposed in a landfill will degrade and emit over a period of decades.

The method does not take into account any landfill gas capture occurring at the landfill.

Table 16 Indirect (scope 3) waste emission factors for total waste disposed to landfill by broad waste stream category

| Waste stream | Scope 3 emission factor (t CO2-e/t) | Volume to mass conversion factor (t/m3) |
| --- | --- | --- |
| Municipal solid waste | 1.6 | 0.36 |
| Commercial and industrial waste | 1.3 | 0.33 |
| Construction and demolition waste | 0.2 | 0.39 |

Source: Derived from National Greenhouse and Energy Reporting (Measurement) Determination 2008. 
Section 5.11

#### Calculating emissions from waste disposal to landfill

Estimates of Scope 3 greenhouse gas emissions associated with the disposal of waste can be calculated as follows:

When weight of waste is known:

t CO2˗e=Q ×EF

When weight is not known:

t CO2˗e= m3 ×CF ×EF

Where:

t CO2-e is the emissions measured in CO2-e tonnes.

Q is the quantity of waste measured in tonnes.

EF is the emission factor of waste type as per Table 15 and Table 16.

m3 is the volume of waste measure in cubic metres.

CF is the conversation factor of volume to mass as per Table 15 and Table 16.

##### Example 9 Calculation of lifetime emissions (t CO2-e) generated from solid waste (t)

A higher education facility produced a total solid waste stream of 240 tonnes which was disposed of in the local landfill.  This waste comprises 140 tonnes of food waste, 50 tonnes of paper/paper board, 10 tonnes of garden and park waste and 40 tonnes of inert waste.  It is assumed that no methane was recovered at the landfill.  The lifetime emissions are estimated as follows:

t CO2˗e=Q ×EF

Where: Q Food waste = 140 tonnes Paper/paper board = 50 tonnes Garden and park waste = 10 tonnes Inert waste = 40 tonnes EF as per Table 15 Food waste = 2.1 t CO2-e/tonne Paper/paper board = 3.3 t CO2-e/tonne Garden and park waste = 1.6 t CO2-e/tonne Inert waste = 0.0 t CO2-e/tonne

Calculation of Total Greenhouse Gas Emissions (t CO2 -e)

Food waste greenhouse gas emissions (t CO2-e) =140 ×2.1 = 294 t CO2-e

Paper/paper board greenhouse gas emissions (t CO2-e) =50×3.3 = 165 t CO2-e

Garden and park waste greenhouse gas emissions (t CO2-e) =10 ×1.6 = 16 t CO2-e

Inert waste greenhouse gas emissions (t CO2-e) =40 ×0.0 = 0 t CO2-e

Total greenhouse gas emissions (t CO2-e) = 475 t CO2-e

##### Example 10 Calculation of lifetime emissions (t CO2-e) generated from commercial and industrial waste

A commercial company in the finance industry disposes 1 kilotonne of commercial and industrial waste. Their lifetime emissions are estimated as follows:

t CO2˗e=Q ×EF

Where: Q = 1000 tonnes EF for Commercial and industrial waste as per Table 16 = 1.3 t CO2-e/tonne

Calculation of Total Greenhouse Gas Emissions (t CO2 -e)

Commercial and Industrial waste greenhouse gas emissions (t CO2-e) =1000 ×1.3 = 1,300 t CO2-e

##### Example 11 Calculation of lifetime emissions (t CO2-e) generated from waste when the weight is unknown, but the volume is known (m3)

A facility does not capture the number of tonnes of waste produced, but they do have data available for the number of collections (24 collections) of 3m3 skips filled and disposed of in the year. The type of waste is food waste. The lifetime emissions are estimated as follows:

t CO2˗e= m3 ×CF ×EF

Where: m3 = 1000 tonnes = 72 m3 of waste per year CF as per Table 15 = 0.50 tonnes of waste/m3 EF as per Table 15 = 2.1 t CO2-e/t of waste

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Total greenhouse gas emissions (t CO2-e) =72 × 0.50 ×2.1 = 75.06 t CO2-e

## Estimating emissions from wastewater treatment

### Emissions from wastewater handling (domestic and commercial)

This section discusses emissions from domestic and commercial wastewater treatment and can be used by those who want to capture the emissions from a known number of people. Companies that own and operate wastewater treatment facilities and wish to calculate a scope 1 emissions estimate for this source, please refer to Chapter 5 – Part 5.3 of the National Greenhouse and Energy Reporting (Measurement) Determination 2008 for guidance.

Table 17 Emission factors by wastewater treatment type

| Wastewater Treatment Method | Emission factors (t CO2-e per person) |
| --- | --- |
| Managed aerobic treatment | - |
| Unmanaged aerobic treatment | 0.1229 |
| Anaerobic digester/reactor | 0.3276 |
| Anaerobic lagoon shallow (<2 metres) | 0.0819 |
| Anaerobic lagoon deep (>2 metres) | 0.3276 |

Source: Derived from IPPC guidelines, see Appendix 5 for further details

#### Calculating emissions from domestic and commercial wastewater

The total quantity of wastewater treated depends on the population that is generating wastewater. The following formula should be used to estimate the emissions (in CO2-e) from domestic wastewater treatment.

t CO2˗e=P ×EF

Where:

t CO2-e is the emissions measured in CO2-e tonnes.

P is the number of people in the population for which emissions are to be estimated.

EF is the emission factor for each wastewater treatment type according to Table 17.

##### Example 12 Calculation of emissions (t CO2-e) from wastewater treatment

A local government area wishes to estimate the emissions from domestic wastewater treatment for a population of 20,000 people, where the treatment is an anaerobic deep lagoon. The emissions are estimated as follows:

t CO2˗e=P ×EF

Where: P = 20,000 people EF as per Table 17 = 0.3276 t CO2-e per person

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Total greenhouse gas emissions (t CO2-e) =20,000 ×0.3276 = 6,552 t CO2-e

## Estimating emissions from waste incineration

Waste incineration is defined as the combustion of solid and liquid waste in controlled incineration facilities. Types of waste incinerated include municipal solid waste, industrial waste, hazardous waste, clinical waste and sewage sludge, which are measured in tonnes.

Scope 3 emissions can be calculated with the scope 1 emission factors provided in Table 18, if for example, a company decides to report on emissions from an off-site incineration activity and has the necessary data. However, these emission factors are specific to the incineration activity and do not include the energy consumed in transporting waste off-site.

Table 18 Direct (scope 1) emission factors for the incineration of waste

| Waste type | Emission factors (t CO2-e/t) |
| --- | --- |
| Clinical Waste | 0.879 |
| Sewage Sludge | - |
| Fossil Liquid | 2.931 |
| Industrial Waste | 1.649 |
| Municipal Solid Waste | 0.0537 |

Source: Derived from the NGER Measurement Determination 2008. See Appendix 5 for further details.

Emissions from the incineration of waste may be estimated according to the following formula.

t CO2˗e = Q × EF

Where:

t CO2-e is the emissions of carbon dioxide released from the incineration of waste type measured in CO2-e tonnes.

Q is the quantity of waste type incinerated in tonnes of wet weight value.

EF is the scope 1 emissions factor in t CO2-e/tonne of waste as per Table 18.

##### Example 13 Calculation of emissions (t CO2-e) from waste incineration

A hospital wishes to estimate the emissions from incinerating 2 tonnes of clinical waste. Using default factors, the emissions are estimated as follows:

t CO2˗e = Q × EF

Where: Q = 2 tonnes of clinical waste EF as per Table 18 = 0.879

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Total greenhouse gas emissions (t CO2-e) =2 ×0.879 =1.758 t CO2-e

## Estimating emissions from the biological treatment of solid waste – composting and anaerobic digestion

Composting and anaerobic digestion of organic waste, such as food waste, garden and park waste and sludge are considered biological treatments of solid waste.

Anaerobic digestion of organic waste expedites the natural decomposition of organic material without oxygen by maintaining the temperature, moisture content and pH close to their optimum values. N2O emissions from anaerobic digestion are assumed to be negligible.

Composting, on the other hand, is an aerobic (with oxygen) process and a large fraction of the degradable organic carbon in the waste material is converted into carbon dioxide (CO2). CH4 is formed in anaerobic sections of the compost, but it is oxidised to a large extent in the aerobic sections of the compost.

Table 19 Direct emission factors (scope 1) for biological treatment of waste

| Waste treatment type | Scope 1 emission factors (t CO2-e/t) |
| --- | --- |
| Composting | 0.046 |
| Anaerobic digestion | 0.028 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, see Appendix 5 for more details.

Emissions from the biological treatment of waste may be estimated according to the following formula:

t CO2˗e = (Q × EF) – R

Where:

t CO2-e is the emissions measured in CO2-e tonnes

Q is the quantity of waste, measured in tonnes, being treated

EF is the scope 1 emissions factor in t CO2-e/tonne of waste as per Table 19.

R is the total amount of CH4 recovered in inventory year, tonnes CO2-e. If unknown, assume zero.

##### Example 14 Calculation of emissions (t CO2-e) from composting

A household has an open-air composting heap to which they contribute approximately 2.5 kg of food waste each week. Using default factors and no methane recovery, the emissions are estimated as follows:

t CO2˗e = (Q × EF) -R

Where: Q 52 weeks x 2.5 kg ÷ 1000 = 0.130 tonnes each year EF as per Table 19 = 0.046 R = 0.00

Calculation of Total Greenhouse Gas Emissions (t CO2-e)

Total greenhouse gas emissions (t CO2-e) = 0.130 ×0.046-0 = 0.006 t CO2-e

# Agriculture

Emissions in the Agriculture sector (IPCC Sector 3) arise from livestock, manure management and crop residue, as well as emissions from rice cultivation, application of nitrogen to soils, and burning of agricultural residues.

## Greenhouse gas emissions from agriculture

Emissions of greenhouse gases are produced on agricultural lands as a result of a number of natural and human-induced processes. These include feed digestion by ruminant livestock, the decay or burning of biomass, the addition of nitrogen- and carbon-containing fertiliser and animal manure, crop residues returned to the soil, nitrogen leaching and runoff, atmospheric deposition, and the anaerobic decomposition of organic matter during flood irrigation.

The principal greenhouse gases estimated for agriculture are methane (CH4) and nitrous oxide (N2O). The main agricultural sources of CH4 are the digestion of feed by livestock and manure management. The main agricultural source of N2O is soils, primarily as a result of the use of nitrogen-based fertilisers on crops and pastures. Manure management is also a source of N2O. Crop residue burning produces some CH4 and N2O, but CO2 emissions are not estimated because it returns to the atmosphere what was recently removed during crop growth. CO2 emissions associated with the application of lime and urea are also accounted for in agriculture.

## Estimating agricultural emissions

State and national-level estimates of greenhouse gas emissions from agriculture are prepared using the methodology set out in the National Inventory Report. Organisations wishing to report emissions from their agricultural operations may draw on this national methodology to make indicative estimates but should note that the methodology uses available nationally-consistent activity data, and these are often regional averages not directly applicable to specific operations dependent on local conditions.

Note that emissions from other on-farm activities may be accounted for in other sections, such as:

Vehicle fuel use (covered in Transport fuels).

The burning of fuels in plant and equipment (covered in Stationary energy emissions).

# Land Use, Land Use Change and Forestry

The Land Use, Land Use Change and Forestry (LULUCF) sector is IPCC sector 4. Emissions and sequestration in this sector arise from activities occurring on forest lands, forests converted to other land uses, grasslands, croplands, wetlands and settlements.

## Greenhouse gas emissions from Land Use, Land Use Change and Forestry

This section covers the estimation of emissions and removals of CO2 associated with activities that change the amount of carbon in the land sector. In the land sector, carbon can either be emitted to the atmosphere, or removed from the atmosphere and stored in plants and soil. Non-CO2 emissions from biomass burning on managed lands is also included.

For example, this sector includes emissions from forest land remaining forest land and land converted to forest land (e.g. harvest and regeneration of native forests, establishment and harvest of plantations, wildfires and prescribed burning), including sinks from regrowing forest on previously cleared land, and carbon stored in harvested wood products and their disposal in landfill.

Another source of emissions in this sector is forest land converted to cropland, grassland, settlements and wetlands, including direct clearing-related emissions and delayed emissions from previous clearing, mainly through the gradual loss of soil carbon over a number of years.

## Estimating Land Use, Land Use change and forestry emissions

The greenhouse accounts for Australia’s land sector reporting use a wide range of spatial data, which include satellite monitoring of changes in land cover and weather records as well as on-ground measurements of trees and soil to calibrate the models. Based on this data, growth of trees and other plants and changes in soil are modelled to estimate carbon stock change and greenhouse gas emissions through time at a fine spatial scale. The model used for Australia’s greenhouse accounts is the Full Carbon Accounting Model (FullCAM).

FullCAM simulates all carbon pools, which include living biomass (trees including roots, grass), dead organic matter and soil. The settings in FullCAM are informed by data on management, site, climate and species, which are drawn from resource inventories, field studies and remote sensing methods.

FullCAM is publicly available from the Department’s website.  The software provides the user with flexibility in entering species parameters and management events which influence carbon stock change. The target audience for the public version of FullCAM is emissions inventory experts, the scientific community, industry, and policy makers with an interest in land sector carbon reporting.

# Glossary

Activity - A process that generates greenhouse gas emissions or uptake. In some sectors it refers to the level of energy consumption, production or manufacture for a given process or category or animal numbers.

Activity data - Source data from a generating activity, such as fuel usage and electricity consumption, and can be used to determine greenhouse gas emissions.

ANZSIC - The Australian and New Zealand Standard Industrial Classification (ANZSIC) is derived from international classifications (International Standard Industrial Classifications (ISIC)) and provides a framework for organising data about businesses - by enabling grouping of business units carrying out similar productive activities. The ANZSIC was developed by the Australian Bureau of Statistics (ABS) in collaboration with Statistics New Zealand.

Carbon dioxide equivalence (CO2-e) - A standard measure that takes account of the global warming potential of different greenhouse gases and expresses the effect in a common unit.

Carbon inventory - A measure of the carbon dioxide equivalent emissions that are attributable to an activity. A carbon inventory can relate to the emissions of an individual, household, organisation, product, service, event, building or precinct. This can also be known as a carbon footprint or carbon account.

Carbon neutral - A situation where the net emissions associated with an activity are equal to zero because emissions have been reduced and offset units cancelled to fully account for all emissions.

Confidentiality - Data that is considered to be commercially sensitive. Confidential emissions are typically reported as an aggregated CO2 equivalent value.

Emission factor - The quantity of greenhouse gases emitted per unit of some specified activity. Emission factors are used to convert a unit of activity into its emissions equivalent. 
E.g. a factor that specifies the kilograms of CO2-e emissions per unit of activity.

Emission Type - The release of a particular gas to the atmosphere as a result of a certain activity. Emissions can be one of the following four types:

Generated - the gross result of a process or activity;

Recovered - the capture of generated emissions for use in a secondary process, such as power generation;

Sinks - a net gain in carbon stocks, such as those found in forests;

Net emissions - the remaining gas released to the atmosphere after generation, recovery and sinks are taken into account.  This is distinct from the emissions accounting concept of net emissions which are those after the application of offsets or other accounting units.

The most common data in the Australia’s National Greenhouse Accounts are net estimates of emissions.

Emissions Projections - Emissions projections detail Australia's greenhouse emissions trends to 2035, analysing the factors driving emissions in each sector and estimating the effort needed to meet Australia's emissions reduction targets.

Emission Source - Any process or activity that releases a greenhouse gas, an aerosol or a precursor of a greenhouse gas into the atmosphere.

Energy content factor for a fuel, means gigajoules of energy per unit of the fuel measured as gross calorific value.

Gas - Rules adopted under the Paris Agreement require parties to report seven greenhouse gases: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), hydrofluorocarbons (HFCs), perfluorocarbons (PFCs), sulphur hexafluoride (SF6) and nitrogen trifluoride (NF3). Emissions from these gases are aggregated into carbon dioxide equivalents (CO2-e) using factors called global warming potentials (GWPs). The default setting for the system is to report emissions of the seven main classes of gases aggregated into a single CO2-e estimate for each sector. Emissions of other, indirect gases, which cannot be aggregated because they do not have GWPs applied to them, are also reported individually. These gases include nitrogen oxides (NOx), carbon monoxide (CO), non-methane volatile organic compounds (NMVOCs) and sulphur dioxide (SO2).

Global Warming Potential (GWP) - Represents the relative warming effect of a unit mass of a greenhouse gas compared with the same mass of CO2 over a specific period. Multiplying the actual amount of gas emitted by the GWP gives the CO2-equivalent (CO2-e) emissions. The GWPs used in the Paris Agreement inventory are from the IPCC Fifth Assessment Report (AR5), consistent with rules agreed under the Paris Agreement.

Oxidation factors is defined as the proportion of carbon contained in a fuel which is oxidised to CO2. Oxidation factors for fuels used in stationary energy are consistent with the IPCC 2006 assumption of complete oxidation of carbon contained in fuel.

# Appendix 1 Units and conversions

Table 20 Metric prefixes

| Abbreviation | Prefix | Symbol | Multiplying factor |
| --- | --- | --- | --- |
| 1015 (106x109) | peta (million billion [thousand trillion]) | p | 1,000,000,000,000,000 |
| 1012 (103x109) | tera (thousand billion [trillion]) | t | 1,000,000,000,000 |
| 109 | giga (billion) | g | 1,000,000,000 |
| 106 | mega (million) | m | 1,000,000 |
| 103 | kilo (thousand) | k | 1,000 |
| 102 | hecto | h | 100 |
| 101 | deca | da | 10 |
| 100 | - (no prefix) | - | 1 |
| 10-1 | deci | d | 0.1 |
| 10-2 | centi | c | 0.01 |
| 10-3 | milli | m | 0.001 |
| 10-6 | micro | µ | 0.000 000 1 |
| 10-9 | nano | n | 0.000 000 000 1 |
| 10-12 | pico | p | 0.000 000 000 000 1 |
| 10-15 | femto | f | 0.000 000 000 000 000 1 |

Table 21 Energy conversion factors

| Unit | Energy conversion factor |
| --- | --- |
| 1 watt | = 1 joule/second |
| 3600 watt-seconds | = 1 watt-hour (3600 seconds in one hour) |
| 1 watt-hour | = 3600 joules |
| 1000 watt-hours | = 1kilowatt hour (kWh) |
| 1 kWh | = 3.6 × 106 joules = 3.6 MJ |
| 1 kwh | = 3.6 × 10-3 GJ |
| 1 GJ | = 278 kWh |
| 1 PJ | = 278 × 106 kWh = 278 GWh |

Example: For conversion from kWh to GJ multiply by 0.0036 and for conversion from GJ to kWh multiply by 277.778

Table 22 Unit equivalences

|  | grams (g) | kilograms (kg) | tonnes (t) | kilotonnes (kt) | megatonne (Mt) | gigatonne (Gt) |
| --- | --- | --- | --- | --- | --- | --- |
| grams (g) | 1 gram= 1 microtonne | 1kilogram = 1,000 g | 1 tonne = 1,000,000 g | 1 kilotonne = 1,000,000,000 g | 1 megatonne= 1,000,000,000,000 g | 1 gigatonne = 1,000,000,000,000,000 g |
| kilograms (kg) | 1 gram= 0.001 kg | 1kilogram = 1 millitonne | 1 tonne = 1,000 kg | 1 kilotonne = 1,000,000 kg | 1 megatonne= 1,000,000,000 kg | 1 gigatonne = 1,000,000,000,000 kg |
| tonnes (t) | 1 gram= 0.000 000 1 t | 1kilogram = 0.001 t | 1 tonne = 1 megagram | 1 kilotonne = 1,000 t | 1 megatonne= 1,000,000 t | 1 gigatonne = 1,000,000,000 t |
| kilotonnes (kt) | 1 gram= 0.000 000 000 1 kt | 1kilogram = 0.000 000 1 kt | 1 tonne = 0.001 kt | 1 kilotonne = 1 gigagram | 1 megatonne= 1,000 kt | 1 gigatonne = 1,000,000 kt |
| megatonne (Mt) | 1 gram= 0.000 000 000 000 1 Mt | 1kilogram = 0.000 000 000 1 Mt | 1 tonne = 0.000 000 1 Mt | 1 kilotonne = 0.001 Mt | 1 megatonne= 1 teragram | 1 gigatonne = 1,000 Mt |
| gigatonne (Gt) | 1 gram= 0.000 000 000 000 000 1 Gt | 1kilogram = 0.000 000 000 000 1 Gt | 1 tonne = 0.000 000 000 1 Gt | 1 kilotonne = 0.000 000 1 Gt | 1 megatonne= 0.001 Gt | 1 gigatonne = 1 petagram |

# Appendix 2 Global Warming Potentials

Global Warming Potentials (GWPs) are used to convert masses of different greenhouse gases into a single carbon dioxide-equivalent metric (CO2-e). This is done by multiplying the quantity of a gas by its GWP in Table 23 to convert relevant non-carbon dioxide gases to a carbon dioxide equivalent (CO2-e). Multiplying a mass of a particular gas by its GWP, gives the mass of carbon dioxide emissions that would produce the same warming effect over a 100-year period.

Table 23 Global Warming Potentials

| Gas | Chemical formula | Global Warming Potential (AR5) |
| --- | --- | --- |
| Carbon dioxide | CO2 | 1 |
| Methane | CH4 | 28 |
| Nitrous oxide | N2O | 265 |
| Sulphur hexafluoride | SF6 | 23,500 |
| HFC-23 (R-23) | CHF3 | 12,400 |
| HFC-32 (R-32) | CH2F2 | 677 |
| HFC-41 (R-41) | CH3F | 116 |
| HFC-43-10mee (R-4310mee) | C5H2F10 | 1,650 |
| HFC-125 (R-125) | C2HF5 | 3,170 |
| HFC-134 (R-134) | C2H2F4 (CHF2CHF2) | 1,120 |
| HFC-134a (R-134a) | C2H2F4 (CH2FCF3) | 1,300 |
| HFC-143 (R-143) | C2H3F3 (CHF2CH2F) | 328 |
| HFC-143a (R-143a) | C2H3F3 (CF3CH3) | 4,800 |
| HFC-152a (R-152a) | C2H4F2 (CH3CHF2) | 138 |
| HFC-227ea (R-227ea) | C3HF7 | 3,350 |
| HFC-236fa (R-236fa) | C3H2F6 | 8,060 |
| HFC-245ca (R-245ca) | C3H3F5 | 716 |
| HFC-245fa (R-245fa) | CHF2CH2CF3 | 858 |
| HFC-365mfc (R-365mfc) | CH3CF2CH2CF3 | 804 |
| HCFC-22 (R-22) | CHClF2 | 1760 |
| HCFC-123 (R-123) | CHCl22CF3 | 79 |
| HCFC-124 (R-124) | CHClFCF3 | 527 |
| HCFC-141b (R-141b) | CH3CCl2F | 782 |
| HCFC-142b (R-142b) | CH2CClF2 | 1980 |
| HCFC-225ca (R-225ca) | CHCl2CF2CF3 | 127 |
| HCFC-225cb (R-225cb) | CHClFCF2CCIF2 | 525 |
| PFC-14 Perfluoromethane (tetrafluoromethane) | CF4 | 6,630 |
| PFC-116 Perfluoroethane (hexafluoroethane) | C2F6 | 11,100 |
| PFC-218 Perfluoropropane | C3F8 | 8,900 |
| PFC-31-10 Perfluorobutane | C4F10 | 9,200 |
| PFC-318 Perfluorocyclobutane | c-C4F8 | 9,540 |
| PFC-41-12 Perfluoropentane | C5F12 | 8,550 |
| PFC-51-14 Perfluorohexane | C6F14 | 7,910 |
| PFC-91-18 Perflunafene | C10F18 | 7,190 |

Source:  For more details of Global Warming Potentials see Chapter 8, Anthropogenic and Natural Radiative Forcing, Appendix 8.A, Table 8.A.1 (page 731) of the IPCC 5th Assessment Report.

Table 24 Blended refrigerants (many containing HFCS and/or PFCS)

| Blend | Constituents | Composition (%) |
| --- | --- | --- |
| R-400 | CFC-12/CFC-114 | R-400 can have various proportions of CFC-12 and CFC-114. The exact composition needs to be specified, e.g., R-400 (60/40). |
| R-401A | HCFC-22/HFC-152a/HCFC-124 | (53.0/13.0/34.0) |
| R-401B | HCFC-22/HFC-152a/HCFC-124 | (61.0/11.0/28.0) |
| R-401C | HCFC-22/HFC-152a/HCFC-124 | (33.0/15.0/52.0) |
| R-402A | HFC-125/HC-290/HCFC-22 | (60.0/2.0/38.0) |
| R-402B | HFC-125/HC-290/HCFC-22 | (38.0/2.0/60.0) |
| R-403A | HC-290/HCFC-22/PFC-218 | (5.0/75.0/20.0) |
| R-403B | HC-290/HCFC-22/PFC-218 | (5.0/56.0/39.0) |
| R-404A | HFC-125/HFC-143a/HFC-134a | (44.0/52.0/4.0) |
| R-405A | HCFC-22/ HFC-152a/ HCFC142b/PFC-318 | (45.0/7.0/5.5/42.5) |
| R-406A | HCFC-22/HC-600a/HCFC-142b | (55.0/14.0/41.0) |
| R-407A | HFC-32/HFC-125/HFC-134a | (20.0/40.0/40.0) |
| R-407B | HFC-32/HFC-125/HFC-134a | (10.0/70.0/20.0) |
| R-407C | HFC-32/HFC-125/HFC-134a | (23.0/25.0/52.0) |
| R-407D | HFC-32/HFC-125/HFC-134a | (15.0/15.0/70.0) |
| R-407E | HFC-32/HFC-125/HFC-134a | (25.0/15.0/60.0) |
| R-408A | HFC-125/HFC-143a/HCFC-22 | (7.0/46.0/47.0) |
| R-409A | HCFC-22/HCFC-124/HCFC-142b | (60.0/25.0/15.0) |
| R-409B | HCFC-22/HCFC-124/HCFC-142b | (65.0/25.0/10.0) |
| R-410A | HFC-32/HFC-125 | (50.0/50.0) |
| R-410B | HFC-32/HFC-125 | (45.0/55.0) |
| R-411A | HC-1270/HCFC-22/HFC-152a | (1.5/87.5/11.0) |
| R-411B | HC-1270/HCFC-22/HFC-152a | (3.0/94.0/3.0) |
| R-411C | HC-1270/HCFC-22/HFC-152a | (3.0/95.5/1.5) |
| R-412A | HCFC-22/PFC-218/HCFC-142b | (70.0/5.0/25.0) |
| R-413A | PFC-218/HFC-134a/HC-600a | (9.0/88.0/3.0) |
| R-414A | HCFC-22/HCFC-124/HC-600a/HCFC-142b | (51.0/28.5/4.0/16.5) |
| R-414B | HCFC-22/HCFC-124/HC600a/HCFC-142b | (50.0/39.0/1.5/9.5) |
| R-415A | HCFC-22/HFC-152a | (82.0/18.0) |
| R-415B | HCFC-22/HFC-152a | (25.0/75.0) |
| R-416A | HFC-134a/HCFC-124/HC-600 | (59.0/39.5/1.5) |
| R-417A | HFC-125/HFC-134a/HC-600 | (46.6/50.0/3.4) |
| R-418A | HC-290/HCFC-22/HFC-152a | (1.5/96.0/2.5) |
| R-419A | HFC-125/HFC-134a/HE-E170 | (77.0/19.0/4.0) |
| R-420A | HFC-134a/HCFC-142b | (88.0/12.0) |
| R-421A | HFC-125/HFC-134a | (58.0/42.0) |
| R-422A | HFC-125/HFC-134a/HC-600a | (85.1/11.5/3.4) |
| R-422B | HFC-125/HFC-134a/HC-600a | (55.0/42.0/3.0) |
| R-422C | HFC-125/HFC-134a/HC-600a | (82.0/15.0/3.0) |
| R-500 | CFC-12/HFC-152a | (73.8/26.2) |
| R-501 | HCFC-22/CFC-12 | (75.0/25.0) |
| R-502 | HCFC-22/CFC-115 | (48.8/51.2) |
| R-503 | HFC-23/CFC-13 | (40.1/59.9) |
| R-504 | HFC-32/CFC-115 | (48.2/51.8) |
| R-505 | CFC-12/HCFC-31 | (78.0/22.0) |
| R-506 | CFC-31/CFC-114 | (55.1/44.9) |
| R-507A | HFC-125/HFC-143a | (50.0/50.0) |
| R-508A | HFC-23/PFC-116 | (39.0/61.0) |
| R-508B | HFC-23/PFC-116 | (46.0/54.0) |
| R-509A | HCFC-22/PFC-218 | (44.0/56.0) |

Source: Table 7.8, Chapter 7: Emissions of Fluorinated Substitutes for Ozone Depleting Substances, 2019 Refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories.

# Appendix 3 Methodology for calculating scope 3 greenhouse gas emissions from liquid and gaseous fuels

The methods used to develop scope 3 emission factors for liquid and gaseous fuels are consistent with the 2006 IPCC Guidelines for National Greenhouse Gas Inventories (IPCC 2006) and are comparable with international practice.

## Gaseous fuels

Scope 3 emissions factors for natural gas have been prepared by Wilkenfeld and Associates (2012) based on data collected under the National Greenhouse and Energy Reporting (NGER) Act 2007. Data associated with the following emission sources reported under the NGER Scheme were incorporated in the scope 3 factors:

| Emission source | Fuel combustion | Fugitive emissions |
| --- | --- | --- |
| Natural gas exploration | Included | Included |
| Natural gas production or processing | Included | Included |
| Natural gas transmission | Included | Included |
| Natural gas distribution | Included | Not included |

## Liquid fuels

Scope 3 emission factors for liquid stationary and transport fuels reflect the emissions associated with fuel produced locally and overseas. Of the petroleum products consumed in Australia, some are refined within Australia from crude oil, whereas others are refined overseas and then imported.

Over the past decade, local production of petroleum products has declined while imports have increased (Figure 1). The source countries for crude oil feedstock and refined products have also varied over time. These changes prompted a review of scope 3 factors for liquid fuels in 2022.

The scope 3 liquid fuel emission factors published in the National Greenhouse Accounts Factors are intended to represent ‘well-to-pump’ emissions.

The calculation method utilised in this analysis separately addresses the following seven components of Australia’s well-to-pump liquid fuel emissions:

Upstream  (including all supply chain emissions prior to refining)

  - Crude oil extracted within Australia for use in Australian refineries

  - Crude oil extracted overseas for use in Australian refineries

  - Crude oil extracted within Australia for use in overseas refineries

  - Crude oil extracted overseas for use in overseas refineries

Midstream (refining)

  - Refining of petroleum products within Australia

  - Refining of petroleum products overseas

Downstream (distribution)

  - Shipping of refined petroleum products from overseas refineries to Australia.

For each component, volume-weighted emissions factors are calculated based on liquid fuel trade data and emission factors derived from Masnadi et al. (2018) and Jing et al. (2020). Full well-to-pump factors are then aggregated by taking the volume-weighted contribution of each of the seven components from well-to-pump. The calculated scope 3 liquid fuel emission factors for Australia range from 17.1 – 20.2 kg CO2-e/GJ, with an average of 18.0 kg CO2-e/GJ. These values are of a similar magnitude to those published in international emissions databases and academic literature as shown in Figure 2.

Figure 1 Imported and Local petroleum products

Figure 2 Emission factors in international databases and academic literature

Data for Figure 2.

LPG

| Reference | Midstream EF | Total EF |
| --- | --- | --- |
| Elgowainey et al, 2014 | 6.6 | 19.6 |
| Sun et al, 2019 | 7.5 | 20.5 |
| PRELIM low | 6.7 | 19.7 |
| PRELIM high | 8.3 | 21.3 |
| DEFRA (UK) |  | 7.2 |
| Base Carbone (EU) |  | 10.1 |
| Point Advisory, 2022 | 7.2 | 20.2 |
| Existing NGA factor |  | 3.6 |
| NGER FY21 (excluding downstream) |  | 13.6 |

Automotive gasoline

| Reference | Midstream EF | Total EF |
| --- | --- | --- |
| Elgowainey et al, 2014 | 7.8 | 18.4 |
| Cooney et al, 2017 | 11.2 | 21.8 |
| Skone et al 2008 | 9.8 | 20.4 |
| Sun et al, 2019 | 7.3 | 17.9 |
| PRELIM low | 8.3 | 18.9 |
| PRELIM high | 13.2 | 23.8 |
| DEFRA (UK) |  | 17.6 |
| Base Carbone (EU) |  | 14.2 |
| Point Advisory, 2022 | 6.6 | 17.2 |

Aviation turbine fuel

| Reference | Midstream EF | Total EF |
| --- | --- | --- |
| Elgowainey et al, 2014 | 2.3 | 13.5 |
| Cooney et al, 2017 | 2.3 | 13.5 |
| Skone et al 2008 | 6.0 | 17.2 |
| Sun et al, 2019 | 1.9 | 13.1 |
| PRELIM low | 2.3 | 13.5 |
| PRELIM high | 2.8 | 14.0 |
| DEFRA (UK) |  | 14.3 |
| Base Carbone (EU) |  | 14.9 |
| Point Advisory, 2022 | 6.8 | 18.0 |

Diesel

| Reference | Midstream EF | Total EF |
| --- | --- | --- |
| Elgowainey et al, 2014 | 4.9 | 15.6 |
| Cooney et al, 2017 | 7.0 | 17.7 |
| Skone et al 2008 | 9.5 | 20.2 |
| Sun et al, 2019 | 4.7 | 15.4 |
| PRELIM low | 4.4 | 15.1 |
| PRELIM high | 13.4 | 24.1 |
| DEFRA (UK) |  | 16.2 |
| Base Carbone (EU) |  | 14.9 |
| Point Advisory, 2022 | 6.6 | 17.3 |

Fuel oil

| Reference | Midstream EF | Total EF |
| --- | --- | --- |
| Elgowainey et al, 2014 | 3.4 | 14.2 |
| Sun et al, 2019 | 3.9 | 14.7 |
| PRELIM low | 0.9 | 11.7 |
| PRELIM high | 9.6 | 20.4 |
| Point Advisory, 2022 | 6.4 | 17.1 |
| Existing NGA factor |  | 3.6 |
| NGER derived (excluding downstream) |  | 13.6 |
| Elgowainey et al, 2014 | 3.4 | 14.2 |
| Sun et al, 2019 | 3.9 | 14.7 |

References

Masnadi, Mohammad S. & El-Houjeiri, Hassan & Schunack, Dominik & Li, Yunpo & Englander, Jacob & Badahdah, Alhassan & Monfort, Jean-Christophe & Anderson, James & Wallington, Timothy & Bergerson, Joule & Gordon, Deborah & Koomey, Jonathan & Przesmitzki, Steven & Azevedo, Inês & Bi, Xiaotao & Duffy, James & Heath, Garvin & Keoleian, Gregory & McGlade, Christophe & Brandt, Adam. (2018). Global carbon intensity of crude oil production. Science. 361. 851-853. 10.1126/science.aar6859.

Jing, Liang & El-Houjeiri, Hassan & Monfort, Jean-Christophe & Brandt, Adam & Masnadi, Mohammad S. & Gordon, Deborah & Bergerson, Joule. (2020). Carbon intensity of global crude oil refining and mitigation potential. 
Nature Climate Change. 10. 1-7. 10.1038/s41558-020-0775-3.

# Appendix 4 Methodology for calculating electricity emission factors

Scope 2 emissions result from purchased or acquired electricity from the grid. The emission factor for scope 2 is defined in terms of energy sent out to the grid rather than energy delivered. There are additional emissions that are attributable to the electricity lost in transmission and distribution which form a component of the scope 3 emission factor.

## Scope 2 data inputs

The Scope 2 emission factor and residual mix factor for the methods outlined in section 2.2 of this report rely on the following data inputs:

Greenhouse gas emissions from fossil fuels from NGER for the latest reporting period (on a financial year basis) - including state-based data and the physical characteristics of the electricity supply and demand.

Interstate trade of electricity from AEMO (sourced through the NEM-Review tool) for the same time period as the NGER emissions data.

Renewable generation from wind, hydro, large scale solar, and small scale solar (residential and commercial rooftops) data from AEMO (sourced through the NEM-Review software tool) for the same time period as the NGER emissions data.

Emissions of methane from water reservoirs used for hydroelectricity generation.

Biofuels calculated using Large-Scale Generation Certificate creations in the Renewable Energy Certificate Register.

- Note that some state-based claims are made using market-based methods, such as the Australian Capital Territory which ensures electricity emissions are zero through its 100% renewable electricity commitment from 2020. The Renewable electricity supply in the ACT is calculated based on the Climate Change and Greenhouse Gas Reduction (Renewable Electricity Target Measurement Method) Determination 2020.

- Care should be taken when comparing such claims to location-based factors – both approaches are equally valid, but not necessarily comparable.

#### Self-consumption

In calculating the scope 2 emission factors, the department excludes electricity generated by a facility that is consumed on site rather than sent to the grid.

Rooftop small solar generation which is self-consumed ‘behind the meter’ is calculated at the national level and is estimated to be 47%, with the remainder (53%) assumed to be sent to the grid. This self-consumption has been deducted from the estimated amount of generation from small solar used to calculate both the location-based method emission factors and the market-based method’s residual mix factor (RMF). The estimate of rooftop solar self-consumption for Australia has been taken from a peer-reviewed paper, Mckenna et al (2019). The estimate of small solar self-consumption cited in McKenna et al (2019) is consistent with sampled data which the Department extracted from PVoutput.org. The Department will update this estimate in future as this independent dataset grows or other data becomes available.

#### Residual Mix Factor for the market-based method

Under the market-based method, the national location-based scope 2 emission factor is adjusted to remove the emissions benefit of all claimable renewable generation through LGCs to produce a RMF. This data comes from the Renewable Energy Certificate (REC) Registry.

LGCs are created on a calendar year basis and may be created up to 12 months after the date of generation. As the factors provided in this workbook are on a financial year basis, a lag percentage is introduced into the calculation to estimate how many LGCs are created after the 6 months covering the date range of the other data inputs. This lag adjustment uses a 3-year average of these “lagged” LGC creations.

To calculate the RMF, LGC creations are deducted from electricity generation sent to the grid. The RMF is calculated at the national aggregate level as the LGC market covers all networks and creations can be made from off-grid generation.

#### Method for calculating the Residual Mix Factor

The RMF is calculated by the Department on the same annual cycle as the location-based method scope 2 emission factors. The calculation method is described below:

RMF = E / (Q – (REC × 1000))

Where:

RMF is the residual mix factor for the financial year, measured in kilograms of CO2-e per MWh.

E is emissions from on-grid generation based on NGER reports for the preceding reporting year, measured in tonnes CO2-e. Emissions associated with cogeneration facilities and those whose primary purpose for electricity is for self-use and not for the grid (for example a self-generating alumina refinery) are pro-rated based on the proportion of the amount of total generation and that which is sent to the grid, as reported under NGER. E includes methane emissions from hydro generation dams (defined as a human-made structure built to contain and control a body of water and used to generate hydroelectricity) based on the method applied in the National Greenhouse Gas Inventory.

Q is electricity generation sent out to the grid, measured in kWh, excluding distributed small-scale solar self-consumption. Generation data is from NGER reports and Australian Energy Market Operator data sourced through the NEMReview software package both for the preceding reporting year.

REC is LGC creations for generation which occurred in the same financial year as E and Q, measured in MWh, and is based on data provided by the CER from the REC Registry.

Notes:

- Grids are the National Electricity Market (NEM), South-West Interconnected System (SWIS), and the Darwin, Katherine Interconnected System (DKIS)

## Transmission and distribution network operators

Companies that own or control transmission and distribution networks report their transmission and distribution loss emissions under scope 2. This follows the GHG Protocol guidance that scope 2 emissions be reported by the organisation owning or controlling the plant or equipment where the electricity is consumed.

Scope 3 emission factors for transmission and distribution network operators include only emissions attributable to the extraction, production and transport of fuels and not emissions attributable to the electricity lost in transmission and distribution networks. These factors, in kg CO2-e/kWh are shown in Table 25.

Table 25 Transmission and distribution losses

| State, Territory or grid description | Transmission and distribution losses kg CO2-e/kWh |
| --- | --- |
| New South Wales and Australian Capital Territory | 0.02 |
| Victoria | 0.08 |
| Queensland | 0.06 |
| South Australia | 0.02 |
| Western Australia - South-West Interconnected System (SWIS) | 0.04 |
| Tasmania | 0.02 |
| Northern territory - Darwin Katherine Interconnected System (DKIS) | 0.03 |
| Western Australia – North-Western Interconnected System (NWIS) | 0.05 |

#### Calculating Scope 2 emission factors for electricity

The emission factor at generation (EFG scope2ti) is used to calculate scope 2 emissions and is defined for state i and financial year t. For the market-based method RMF estimation, all inputs are aggregated to the national level.

EFG scope2it=Combustion emissions from electricity consumed from the grid in state i CECitElectricity sent out consumed from the grid in state i ESOCit

Where:

‘Combustion emissions from electricity consumed from the grid in state i’ (CE_Cti) and ‘energy sent out consumed from the grid in state i’ (ESO_Cti) are defined in terms of the state’s electricity grid generation, imports and exports as follows:

CE_Cit=CE_Pit+jESO_Mj,itESO_Pjt×CE_Pjt-kESO_Xi,ktESO_Pit×CE_Pit

ESO_Cit=ESO_Pit+jESO_Mj,it-kESO_Xi,kt

Where:

CE_Pti is the total CO2-e emissions from fuel combustion for generation attributed to the electricity generated/produced for the grid in state i in financial year t.

CE_Ptj is the total CO2-e emissions from fuel combustion for generation attributed to the electricity generated for the grid in state j in financial year t.

ESO_Mtj,i is the imports of energy sent out from state j to state i in financial year t. Imports are calculated from the interregional flows of electricity across the interconnectors.

ESO_Xti,k is the exports of energy sent out from state i to state k in financial year t. Exports are calculated from the inter-regional flows of electricity across the interconnectors.

ESO_Ptj is the total energy sent out on the grid that is generated within state j in financial year t.

ESO_Pti is the total energy sent out on the grid that is generated within state i in financial year t.

Note:

- The estimated electricity emission factors have been aligned with the definitions used in The Greenhouse Gas Protocol: A Corporate Accounting and Reporting Standard of the World Resources Institute/World Business Council for Sustainable Development (the GHG Protocol).

# Appendix 5 Methodology for calculating greenhouse gas emissions from waste

## Calculating emission factors for wastewater treatment

Table 26 Simplification of emissions equations for wastewater treatment methods

| Wastewater Treatment Method | Methane correction factor (MCFw) | Default Chemical Oxygen Demand (t/person/ year) (DCw) | Default methane emissions in wastewater (t CO2-e/t COD) (EFw) | Emission factor (t CO2-e/person) (EF) |
| --- | --- | --- | --- | --- |
| Managed aerobic treatment | 0 | 0.0585 | 7 | - |
| Unmanaged aerobic treatment | 0.3 | 0.0585 | 7 | 0.1229 |
| Anaerobic digester/reactor | 0.8 | 0.0585 | 7 | 0.3276 |
| Anaerobic lagoon shallow (<2 metres) | 0.2 | 0.0585 | 7 | 0.0819 |
| Anaerobic lagoon deep (>2 metres) | 0.8 | 0.0585 | 7 | 0.3276 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 5.25, Chapter 5, Part 5.3, Division 5.3.2

- Note: Emission factors constructed using EF= DCw ×EFw ×MCFw

## Calculating emission factors for waste incineration

Table 27 Simplification of emissions equations for waste incineration

| Waste Type | Carbon Content (CC) | Fossil Origin (FCC) | Default Oxidation Factor (OF) | Convert to t CO2-e | Emission factor t CO2-e (EF) |
| --- | --- | --- | --- | --- | --- |
| Clinical Waste | 0.6 | 0.4 | 1 | 3.667 | 0.88 |
| Sewage Sludge | 0.5 | - | 1 | 3.667 | - |
| Fossil Liquid | 0.8 | 1.0 | 1 | 3.667 | 2.93 |
| Industrial Waste | 0.5 | 0.9 | 1 | 3.667 | 1.65 |

- Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 5.51, Chapter 5, Part 5.5, and Section 5.4.1 Volume 5, Chapter 5 2006 IPCC guidelines.

- Note: Emission factors constructed using EF= CCi × FCCi × OFi × 3.667

Table 28 Simplification of emissions equations for waste incineration - municipal waste

| Waste types in the Municipal Solid Waste broad stream | Proportion of waste stream (%) | Carbon Content (CC) | Fossil Origin (FCC) | Default Oxidation Factor (OF) | Convert to CO2 | Weighted Emission factor t CO2-e (EF) |
| --- | --- | --- | --- | --- | --- | --- |
| Food | 35.0% | 38% | 0% | 1 | 3.667 | - |
| Paper | 13.0% | 46% | 1% | 1 | 3.667 | 0.0022 |
| Garden and Green | 16.5% | 49% | 0% | 1 | 3.667 | - |
| Wood | 1.0% | 50% | 0% | 1 | 3.667 | - |
| Textiles | 1.5% | 50% | 20% | 1 | 3.667 | 0.0055 |
| Sludge | 0.0% | 50% | 0% | 1 | 3.667 | - |
| Nappies | 4.0% | 70% | 10% | 1 | 3.667 | 0.0103 |
| Rubber and Leather | 1.0% | 67% | 20% | 1 | 3.667 | 0.0049 |
| Other | 28.0% | 3% | 100% | 1 | 3.667 | 0.0308 |

- Weighted Emission factor t CO2-e = 0.0537

- Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 5.11, Chapter 5, Part 5.2, division 5.2.2 and Section 2.3.1 Volume 5, Chapter 2 2006 IPCC guidelines.

- Note: Municipal solid waste incineration emission factor constructed using EF= % ×CCi × FCCi × OFi × 3.667

## Calculating emission factors for the biological treatment of waste

Table 29 Simplification of emissions equations for biological treatment of waste

| Biological treatment type | CH4 emission factor (CO2-e) | N2O emission factor (CO2-e) | Scope 1 emission factor (CO2-e) |
| --- | --- | --- | --- |
| Composting | 0.021 | 0.025 | 0.046 |
| Anaerobic Digestion | 0.028 | 0 | 0.028 |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 5.22, Chapter 5 Waste, Part 5.2, Division 5.2.6

Note: Emission factors constructed using EF= CH4 EF+N2O EF

Table 30 Simplification of emissions equations for solid waste disposal

| Variable | Description and default values |
| --- | --- |
| Q (Activity) | Quantity of municipal solid waste expressed in tonnes and sourced from waste records or contractor invoices. |
| DOC | Degradable Organic Carbon expressed as a proportion of the particular waste type with default values as follows: - Food – 0.15 - Paper and cardboard – 0.4 - Garden and green – 0.2 - Wood – 0.43 - Textiles – 0.24 - Sludge – 0.05 - Nappies – 0.24 - Rubber and leather – 0.39 - Inert waste (including concrete/metal/plastics/glass) - 0 Alternative waste treatment residues – 0.08 |
| DOCF | Fraction of Degradable Organic Carbon dissimilated for the waste type produced with default values as follows: - Food – 0.84 - Paper and cardboard – 0.49 - Garden and green – 0.47 - Wood – 0.10 - Textiles – 0.5 - Sludge – 0.5 - Nappies – 0.5 - Rubber and leather – 0.5 - Inert waste, including concrete, metal, plastic and glass – 0.0 Alternative waste treatment residues – 0.5 |
| F1 | Methane fraction of landfill gas which has a default value of 0.50. |
| 1.336 | Conversion rate of carbon to methane. |
| R | The quantity of recovered methane during the year, measured and expressed in tonnes. Note: Emission factors in Table 15 do not take into account any landfill gas capture occurring at the landfill. If measured, recovered methane must be subtracted from methane generated before applying the oxidation factor. Only the landfill gas that is not captured is subject to oxidation. |
| OX | Oxidation factor which has a default value of 0.1 for covered, well-managed landfills (and a value of 0 for uncovered landfills). |
| 28 | CH4 global warming potential used to convert the quantity of methane emitted to CO2-e from the quantity of waste produced. |

Source: National Greenhouse and Energy Reporting (Measurement) Determination 2008, Section 5.4, Chapter 5, Part 5.2, Division 5.2.2

# Appendix 6 Further Information

| Programs and initiatives | Additional Information |
| --- | --- |
| Clean Energy Regulator | The Clean Energy Regulator (CER) is an Australian independent statutory authority responsible for implementing legislation to reduce carbon emissions and increase the use of clean energy. The CER manages the Large-Scale Renewable Energy Target, which aims to incentivise the development of renewable energy power stations by meeting the annual target for renewable electricity set out in the Renewable Energy (Electricity) Act 2000 (the Act). The annual target increased each year until 2020 and is now constant at 33,000,000 megawatt hours (MWh) to 2030. The Renewable Power Percentage used in scope 2 market-based method, is determined from the target of renewable energy and total acquisitions. |
| Climate Active | Climate Active is an ongoing partnership between the Australian Government and Australian businesses to drive voluntary climate action. Climate Active certifies business that have credibly reached a state of carbon neutrality by measuring, reducing and offsetting their carbon emissions. Certification is available for business operations, products and services, buildings, events and precincts. You can find Certified Brands on the Climate Active website. The Tools and Resource section contains the Climate Active Standards, technical guidance, and other useful links and guidance for estimating emissions. |
| Green Vehicle Guide (GVG) | The GVG helps consumers by providing user friendly tools to search for and compare the environmental performance and fuel consumption of new light vehicles (up to 3.5 tonnes gross vehicle mass) sold in Australia since 2004. It uses the CO2 emissions values for each light vehicle as the key measure for ranking and comparing all light vehicles |
| Energy.gov.au | Energy.gov.au provides a starting point for energy information from a range of Australian Government sources. It provides information and advice for householders and businesses as well as explaining Australian Government energy programs and priorities. |
| Equipment Energy Efficiency (E3) program | The E3 Program helps improve the energy efficiency of a range of appliances and equipment. It is a shared initiative of the Australian Government, states and territories and the New Zealand Government. The program sets out the minimum energy efficiency standards and energy rating labelling requirements for products like fridges, dishwashers, TV’s and more. By choosing more efficient models, households and businesses can consume less energy, save money on their energy bills, and help to reduce greenhouse gas emissions. |
| Your Home | Your Home is Australia’s independent guide to designing, building or renovating homes to ensure they are energy efficient, comfortable, affordable and adaptable for the future. |
| Nationwide House Energy Rating Scheme (NatHERS) | NatHERS measures a home’s energy efficiency to generate a star rating. The higher the star rating, the less energy needed to heat and cool the home to keep it comfortable. NatHERS Assessors currently use the house plans and building specifications of a home to input data into a NatHERS accredited software tool. NatHERS tools estimate the amount of heat that needs to be added or removed to keep that home comfortable. The NatHERS tools then generate a NatHERS star rating out of 10 and a Certificate. This star rating measures the home’s thermal performance, based on its structure, design and materials. |
| National Australian Building Environment Ratings System (NABERS) | NABERS is a national rating system that measures the environmental performance of Australian buildings and tenancies. NABERS measures the energy efficiency, water usage, waste management and indoor environment quality of a building or tenancy and its impact on the environment. |

# Appendix 7 NGER legislative amendments for the 2024–25 reporting year

Amendments have been made to the National Greenhouse and Energy Reporting (NGER) scheme legislation. Public consultation took place from 29 April 2024 to 24 May 2024. These amendments:

- introduce a staggered phase out of the use of Method 1 for estimating fugitive methane emissions from the extraction of coal from open-cut mines covered by the Safeguard Mechanism from 1 July 2025.

- add a Method 2B for the estimation of fugitive methane and carbon dioxide emissions from flaring of gas in natural gas production, providing a mass balance approach.

- update Method 2 for the estimation of fugitive methane emissions from produced formation water occurring in oil or gas operations, to reflect onsite operations where the water has not been exposed to atmosphere and where facilities capture and recycle methane back into the gathering line rather than allowing it to dissolve into the resource pond and leak into the atmosphere.

- correct the categorisation of specified instances of Method 1 for estimating emissions of methane from natural gas venting activities that are consistent with Method 2 requirements.

- update Method 2 for the estimation of fugitive emissions of greenhouse gases from the injection of a greenhouse gas into a geological formation to align with Method 2 for onshore natural gas production.

- reinstate a method for estimating emissions of methane from mud de‑gassing activities during oil or gas exploration and development.

- enable the market-based reporting of scope 1 emissions from renewable liquid fuels when they are co-mingled with their fossil fuel equivalents and supplied through shared infrastructure.

- enable identification of circumstances in which there exists and overlap between a company’s reported scope 1 and scope 2 emissions.

- introduce a reporting requirement for landfills reporting over 100 kilotonnes of carbon dioxide equivalent to provide an estimate of gross emissions from non-legacy waste.

More information is available on the Clean Energy Regulator Measurement Determination webpage.

These amendments took effect on 1 July 2024 and affect reports due by 31 October 2025 for the 2024–25 reporting year.

For additional information, please contact the Clean Energy Regulator.

Factors represent average annual leakages from operation and not leakages during installation or disposal of equipment. Equipment charged with HFC refrigerants should be disposed of in accordance with applicable regulatory requirements.

While emissions of R22 (HCFC-22) are not counted in national inventories submitted under the United Nations Framework Convention on Climate Change and Paris Agreement, its Global Warming Potential is presented here for completeness.

GWP values presented are not limited to gases reportable in national inventories and include Montreal Protocol gases.